APPROVAL PAGE

| Generic Name: Dextrose 5% w/v (INTRAVENOUS INJECTION) B.P. Each 100 ml Contains: Dextrose B.P 5.0gm Water for injection USP. q.s. Packing Size: 1000ml in a PVC pouch. Shelf Life: 3 years, proceeding date of production. |  |
| --- | --- |
| PRODUCT MASTER FORMULA | ## BML/QA/MFR/004 |
| BMR CODE. No.: | BML/BMR/QA/004 |
| Review date: | January, 2026 |
| Next Review Date: | January, 2028 |

| Document Issued by: Sign/Date: | Collected by: Sign/Date: |
| --- | --- |

Batch to be manufactured according to the Product Master Formula Number: BML/QA/MFR/004 without any modification / with the following modifications.

______________________________

Authorized by / Date (Superintendent Pharmacist)

TABLE OF CONTENT

| S/n | Title | Page number |
| --- | --- | --- |
|  | APPROVAL PAGE | 1 |
|  | TABLE OF CONTENTS | 2 |
|  | MATERIAL REQUISITION | 3 |
|  | PRODUCTION EQUIPMENT LIST | 4 |
|  | MANUFACTURING CHECK LIST | 6 |
|  | WEIGHING RECORDS | 7 |
|  | DISPENSING CHECKLIST/CLEARANCE | 7 |
|  | DISPENSING ROOM STATUS | 7 |
|  | APPROVAL TO START DISPENSING OPERATION | 7 |
|  | ACTIVE INGREDIENT IDENTIFICATION | 7 |
|  | PVC POUCH DISPENSING FOR CODING | 8 |
|  | BATCH CODING LINE CLEARANCE | 8 |
|  | CODING SETTING AND IMPRESSION | 8 |
|  | APPROVED POUCH CODING SPECIMEN ATTACH | 10 |
|  | RECONCILIATION OF POUCH CODING ACTIVITIES | 11 |
|  | MIXING ROOM CHECKLIST | 11 |
|  | MIXING LINE CLEARANCE MONITORING FORM | 12 |
|  | MIXING INSTRUCTION PROCEDURE | 13 |
|  | IN-PROCESS SAMPLE ANALYSIS INTIMATION AND REPORT | 14 |
|  | FILLING LINE CLEARANCE | 15 |
|  | FILLING RECORDS | 16 |
|  | PERIODIC VOLUME CHECK PAGE | 17 |
|  | RECONCILIATION OF FILLING PROCESS | 17 |
|  | STERILIZATION RECORDS PAGE | 18 |
|  | PACKAGING RECORDS/CHECKLIST | 19 |
|  | PACKAGING INSTRUCTIONS | 20 |
|  | PACKAGING RECONCILIATION | 20 |
|  | GENERAL RECONCILIATION | 21 |
|  | CHECK LIST OF BATCH RECORDS | 22 |
|  | DECLARATION PAGE AND PRODUCT RELEASE | 22 |

| MATERIALS REQUISITION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Product Name: Dextrose 5% Injection |  |  |  |  |
| Raw Materials Requisition |  |  |  |  |
| Raw Material Name | Material code | QC A. R. No. | Quantity Requested | Quantity Issued |
| Dextrose Monohydrate/Dextrose Anhydrous | RMA0002 |  |  |  |

| Requested By Production Pharmacist | Issued By Store officer | Checked By IPQA |
| --- | --- | --- |
| Sign/ Date | Sign/ Date | Sign/ Date |

| Packaging Materials Requisition |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Packaging Material | Material code | QC A. R. No. | Quantity Requested | Quantity Issued | Quantity used |
| PVC Pouches (1000ml) | PKF0007 | PM/AR/QC/ |  |  |  |
| Polycarbonate Stoppers | PKF0002 | PM/AR/QC/ |  |  |  |
| Wrapping Nylons | PKF0003 | PM/AR/QC/ |  |  |  |
| Mono-cartons | PKF0004 | PM/AR/QC/ |  |  |  |
| Labels | PKF0005 | PM/AR/QC/ |  |  |  |
| Shippers Cartons | PKF0006 | PM/AR/QC/ |  |  |  |

| Requested By Production Pharmacist | Issued By Store officer | Checked By IPQA |
| --- | --- | --- |
| Sign/ Date | Sign/ Date | Sign/ Date |

PRODUCTION EQUIPMENT LIST

| Equipment Description | Machine Code | Sop Operation Number | Sop Cleaning Number | Equip. used | Cleaned/ Cal. by | Checked By |
| --- | --- | --- | --- | --- | --- | --- |
| LOCATION: SOLUTION PREPARATION ROOM |  |  |  |  |  |  |
| Digital Balance (15kg) | BL/SPR/PE/001 | BML-QDG-QA-079 | BML-PDF-PR-006 |  |  |  |
| Electronic Balance (50kg) | BL/SPR/PE/002 | BML-QDG-QA-079 | BML-PDF-PR-006 |  |  |  |
| Dispensing Booth | BL/DSB/PE/001 | BML-PDF-PR-015 | BML-QDG-QA-064 |  |  |  |
| Mixing Vat 1 (2,000 litres) | BL/SPR/PE/007-1 | BML-PDF-PR-007 | BML-PDF-PR-007 |  |  |  |
| Mixing Vat 2 (2,000 litres) | BL/SPR/PE/007-2 | BML-PDF-PR-007 | BML-PDF-PR-007 |  |  |  |
| Mixing Vat 3 (2,000 litres) | BL/SPR/PE/007-3 | BML-PDF-PR-007 | BML-PDF-PR-007 |  |  |  |
| Pass box 1 | BL/FR/PE/001 | BML-PDF-PR-029 | BML-PDF-PR-029 |  |  |  |
| LOCATION: FILLING ROOM |  |  |  |  |  |  |
| Semi Automatic Filling Machine 1 | BL/FR/PE/001 | BML-PDF-PR-010 | BML-PDF-PR-010 |  |  |  |
| Semi Automatic Filling Machine 2 | BL/FR/PE/002 | BML-PDF-PR-010 | BML-PDF-PR-010 |  |  |  |
| Semi Automatic Filling Machine 3 | BL/FR/PE/003 | BML-PDF-PR-010 | BML-PDF-PR-010 |  |  |  |
| Semi Automatic Filling Machine 4 | BL/FR/PE/004 | BML-PDF-PR-010 | BML-PDF-PR-010 |  |  |  |
| Semi Automatic Filling Machine 5 | BL/FR/PE/005 | BML-PDF-PR-010 | BML-PDF-PR-010 |  |  |  |
| Pass Box 2 | BL/FR/PE/002 | BML-PDF-PR-029 | BML-PDF-PR-029 |  |  |  |
| Pass Box 3 | BL/FR/PE/003 | BML-PDF-PR-029 | BML-PDF-PR-029 |  |  |  |
| LOCATION: BATCH CODING ROOM |  |  |  |  |  |  |
| Batch Coding Machine 1 | BL/BC/PE/001 | BML-PDF-PR-020 | BML-PDF-PR-020 |  |  |  |
| Batch Coding Machine 2 | BL/BC/PE/002 | BML-PDF-PR-020 | BML-PDF-PR-020 |  |  |  |
| Batch coding machine (S.A.1) | BL/BC/PE/003 | BML-PDF-PR-020 | BML-PDF-PR-020 |  |  |  |

| Batch coding machine (S.A.2) | BL/BC/PE/004 |  | BML-PDF-PR-020 |  |  | BML-PDF-PR-020 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pass Box 4 |  | BL/FR/PE/002 |  |  | BML-PDF-PR-029 |  |  | BML-PDF-PR-029 |  |  |  |  |  |  |
| LOCATION: AUTOCLAVING AREA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| De Lama Autoclave |  | BL/AC/PM/001 |  |  | BML-PDF-PR-017 |  |  | BML-PDF-PR-017 |  |  |  |  |  |  |
| Pharmalab Autoclave |  | BL/AC/PM/002 |  |  | BML-PDF-PR-018 |  |  | BML-PDF-PR-018 |  |  |  |  |  |  |
| Production Pharmacist/Supervisor |  |  |  | Date |  |  | Quality Assurance Officer |  |  |  |  |  |  | Date |

| Equipment Description | Machine Code | Sop Operation Number | Sop Cleaning Number | Equip. used | Cleaned/ Cal. by | Checked By |
| --- | --- | --- | --- | --- | --- | --- |
| LOCATION: PACKAGING UNIT |  |  |  |  |  |  |
| Leak testing machine 1 | BL/LKM/PE/001 | BML-PDF-PK-035 | BML-PDF-PK-035 |  |  |  |
| Leak testing machine 2 | BL/LKM/PE/002 | BML-PDF-PK-035 | BML-PDF-PK-035 |  |  |  |
| Sighting board 1 | BL/SB/PE/001 | BML-PDF-PK-027 | BML-PDF-PK-027 |  |  |  |
| Sighting board 2 | BL/SB/PE/002 | BML-PDF-PK-027 | BML-PDF-PK-027 |  |  |  |
| Overwrapping machine 1 | BL/SWM/PE/001 | BML-PDF-PK-046 | BML-PDF-PK-045 |  |  |  |
| Overwrapping machine 2 | BL/SWM/PE/002 | BML-PDF-PK-046 | BML-PDF-PK-046 |  |  |  |
| Others1 |  |  |  |  |  |  |
| Others2 |  |  |  |  |  |  |
| Packaging Supervisor | Date | Quality Assurance Officer |  |  |  | Date |

| MANUFACTURING CHECKLIST |  |  |  |  |
| --- | --- | --- | --- | --- |
| - General Safety Procedures - All staff should clean hands with antiseptic liquid soap, use disinfectants and dry them using hand dryers as per sop proper use of lavatory (BML-QDG-QA-008) - All staff should wear face masks, hand gloves, factory overalls and clean rubber shoes as per sop on gowning and use of change room (BML-QDG-QA-011) |  |  |  |  |
| - General Good Documentation Practice - All manufacturing process activities should be recorded in the BMR - All documentation work should be recorded online and properly - All entries in the BMR should be legible and clear with no overwriting. If any mistake cross with a single line and rewrites the correct form, sign the error as per sop on document control and management (BML-QDG-QA-002) |  |  |  |  |
| - Cleaning/Sterilization - Ensure that all machinery and equipment have been cleaned according to their respective SOP’s | Remark (Yes/No) | Comments (ok/Not ok) | Checked by | Verified by |
| - Materials Availability - Ensure sufficient demineralised water is available for cooling of Autoclave |  |  |  |  |
| - Ensure sufficient WFI water is available for manufacturing of the batch |  |  |  |  |
| - Ensure sufficient filters (0.8 whatman, 0.45 and 0.2 Micron filters) is available for manufacturing of the batch |  |  |  |  |
| - Ensure that QC test materials and chemicals are available for in-process analysis |  |  |  |  |
| - Ensure that primary packaging materials are available |  |  |  |  |

| WEIGHING RECORDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| DISPENSING LINE CLEARANCE |  |  |  |  |  |
| s/n | Checks | Remark (Yes/No) | Comments (ok/Not ok) | Checked by | Verified by |
|  | Dispensing room and Dispensing booth cleaned? |  |  |  |  |
|  | Balance/Scale verified? |  |  |  |  |
|  | Scoops, Spatulas, Weighing Balance verified? |  |  |  |  |
|  | Verification of scales and balances done? |  |  |  |  |
|  | Waste from the previous product has been cleared? |  |  |  |  |
|  | Floors, Walls, Pass Box been clean/cleared |  |  |  |  |
|  | Papers, Tags, Forms needed affixed properly? |  |  |  |  |

| DISPENSING ROOM STATUS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ROOM | CONDITIONS |  |  |  |  |
|  | TEMPERATURE NMT 25oC | RELATIVE HUMIDITY NMT 75% | DIFFERENTIAL PRESSURE NLT 1mmH20 | VERIFIED BY | REMARKS |
| DISPENSING BOOTH |  |  |  |  |  |

| APPROVAL TO START DISPENSING OPERATION |  |  |
| --- | --- | --- |
| IPQA Analyst Yes No |  |  |
| Sign: | Date: | Time: |

| QC ACTIVE INGREDIENT IDENTIFICATION |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Material |  |  | Specification |  |  |  |  | Result |  |
| Dextrose Monohydrate BP |  | Stp ref. no |  |  | BML/QA/STP/RMA/002 |  |  |  |  |
| Description of Materials | Q.C AR. Number |  | Quantity Weighed |  |  | Unit (Kg) | Weighed by | Checked by | Verified by QA |
| Dextrose Monohydrate B.P. |  |  | Std QTY | 99.00 |  |  |  |  |  |
|  |  |  | Net |  |  |  |  |  |  |
| Production Pharmacist/Supervisor |  |  | Date |  |  | Q. A. Officer |  |  | Date |

| PVC POUCH DISPENSE FOR CODING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description of Materials | Q.C AR. Number | Quantity Required | Quantity Issued | Checked by | Verified by QA |
| 500ml PVC Pouches |  | 1750Pouches |  |  |  |
| Polycarbonate Stoppers |  | 1750 stoppers |  |  |  |

| BATCH CODING LINE CLEARANCE |  |  |
| --- | --- | --- |
| Previous Product : | B.No: |  |
| Present Product: | B.No: |  |
| Check List | Checked by (Production) | Verified by (QA ) |
| Area Cleanliness (Floor, walls, ceiling, Machines, Pass box). |  |  |
| Ensure Proper pouches are taken for coding. |  |  |
| Ensure Proper Coder setting is printed on the pouches. |  |  |
| Environmental Conditions : Std Temp. : NMT 27°C, Actual Temp. : 0C |  |  |

| CODING SETTING AND IMPRESSION |  |
| --- | --- |
| Coder setting and Impressions given by | Name: Sign/Date: |
| Clarity |  |
| Batch No Setting |  |
| Mfg. Setting |  |
| Exp. Setting |  |

| CHECKED BY: Production Pharmacist/Supervisor | VERIFIED BY: QA Officer |
| --- | --- |
| Sign & Date: | Sign & Date: |

| CHECKLIST |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Check the cleanliness of individual workstations, and the environment is adequately termed “clean” |  |  |  |  |  |
| Ensure all operators are properly kitted |  |  |  |  |  |
| Inspect each code station to conformity to product, batch and period. |  |  |  |  |  |
| Ensure coding is adequately filled in the print area. Repeat procedures 15 minutes from the commencement of the batch. |  |  |  |  |  |
| Time | Checked by | Verified by | Time | Checked by | Verified by |

Pouch Coding: First pouch impression setting to be checked by Production Pharmacist/Supervisor verified by Q.A. Officer & retained in the BMR.

Specimen of Pouch After Setting (Production Supervisor and IPQA personnel shall verify and Sign the Specimen Label) Production Pharmacist/Supervisor IPQA officer sign/date: …………………… sign/date: ……..……………

| RECONCILIATION OF POUCH CODING ACTIVITY |  |
| --- | --- |
| ITEM/ PARAMETER | PVC POUCH |
| Qty. Received |  |
| Qty. Printed |  |
| Qty. Rejected |  |
| Unprinted Qty. Return to store |  |
| Quantity submitted for approval |  |
| Percentage yield(%) |  |
| Batch coding done by |  |
| Checked by (Production Supervisors): |  |
| Verified by (QA): |  |

| MIXING ROOM CHECKILIST |  |
| --- | --- |
| s/n | Instructions |
| 1. | Ensure that the area, equipment and accessories are clean. All remnant material of previous batch is removed. |
| 2. | Line clearance is taken before starting the mixing operation. |
| 3. | Gloves, nose masks are to be worn during mixing. |
| 4. | Environmental conditions are to be recorded properly. |
| 5. | All weighed materials must be properly labelled |
| 6. | Ensure WFI is cleared by QC before mixing |
| 7. | Carry out the mixing operation according to cGMP |

| Previous Product: B. NO.: |  |  |
| --- | --- | --- |
| Check List | Checked by(Production) | Verified by (QA) |
| Area Cleanliness |  |  |
| Equipment &accessories (Mixing tank, stirrer, transfer pumps etc.) cleanliness |  |  |
| Absence of previous product / batch materials |  |  |
| Personnel Accessories (PPE and cleanliness) |  |  |

| MIXING LINE CLEARANCE MONITORING FORM |  |  |  |  |
| --- | --- | --- | --- | --- |
| Time: | Clean |  | Not clean |  |
| Area |  |  |  |  |
| Personnel |  |  |  |  |
| Equipment/Vessel | Equipment/Vessel No. | Cleaned by | Status | Inspected by QA |
| Weighing Balance | BL/SPR/PE/002 |  |  |  |
| Lamina Air flow ( Dispensing Booth ) | BL/DSB/PE/001 |  |  |  |
| Mixing Vessel ( 2000litres ) | BL/SPR/PE/007 |  |  |  |
| Present / Absent of previous product | Parameters |  | Specification | Result |
| Previous product: | Colour |  | colourless |  |
| Batch No: | Identification |  | Must comply |  |
| Present product: | pH |  | 5.0-7.0 |  |
| Batch No: | Conductivity |  | NMT 1.3µs |  |
| Remark : |  |  | QA Officer | Date |

MIXING INSTRUCTIONS PROCEDURE

| Date: |  | Time Started | Time Stopped | Done by Operator | Checked by Supervisor | Verified by QA |
| --- | --- | --- | --- | --- | --- | --- |
| 1. | Transfer 1400 Liters of WFI required into the appropriate mixing vat |  |  |  |  |  |
| 2. | Switch on the stirrer and add 99.00kg Dextrose monohydrate gradually until the total quantity is transferred into the water. Stir for 10 minutes |  |  |  |  |  |
| 3. | Stir until an homogenous solution is obtained |  |  |  |  |  |
| 4. | Switch off the stirrer and make the volume of the solution with sterile apyrogenic water to the batch volume |  |  |  |  |  |
| 5. | Switch on the stirrer and stir for 10 minutes |  |  |  |  |  |
| 6. | Request for the QC department to take a sample for in-process analysis |  |  |  |  |  |
| 7. | On QC department release, switch off the stirrer |  |  |  |  |  |
| 8. | Transfer the solution to the filling room through the Millipore filtration system |  |  |  |  |  |
| 9. | N.B. Use 0.8 whatman filter as pre-filter, then 0.45and 0.2 micron filters for final filtration & record number of filter pads used. |  |  |  |  |  |
| Time Process completed: |  |  |  |  |  |  |
| Production Pharmacist/Supervisor |  | Date |  | Q.A. Officer |  | Date |

IN – PROCESS SAMPLE ANALYSIS INTIMATION AND REPORT

| Product: Dextrose 5% solution |  |  | Batch No.: DL2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Time given: |  |  | Date: |  |  |  |  |
| Intimation given by: |  |  | Received by: |  |  |  |  |
| Sampled by: |  |  | Release Time: |  |  |  |  |
| STP Ref no: BML/QA/STP/FPF/006 |  |  |  |  |  |  |  |
| QC Equipment Status |  |  |  |  |  |  |  |
| Equipment Description | Machine Code | Sop Operation No. |  | Sop Cleaning No. | Equip. used | Equip. Status | Checked By |
| Digital PH Meter | BL/EQ /QAE/007 | BML-QDF-CL-009 |  | BML-QDF-CL-009 |  |  |  |
| Hand-held PH | BL/EQ /QAE/010 | BML-QDF-CL-008 |  | BML-QDF-CL-008 |  |  |  |
| Digital Conductivity Meter | BL/EQ /QAE/008 | BML-QDF-CL-010 |  | BML-QDF-CL-010 |  |  |  |
| Hand-held Cond. meter | BL/EQ /QAE/009 | BML-QDF-CL-028 |  | BML-QDF-CL-028 |  |  |  |
| Auto. Polarimeter | BL/EQ /QAE/002 | BML-QDF-CL-027 |  | BML-QDF-CL-027 |  |  |  |
| Analogue Polarimeter | BL/ EQ/QAE/003 | BML-QDF-CL-013 |  | BML-QDF-CL-013 |  |  |  |
| UV Spectrophotometer | BL/EQ /QAE/001 | BML-QDF-CL-011 |  | BML-QDF-CL-011 |  |  |  |
| Fume cupboard | BL/EQ /QAE/013 | BML-QDF-CL-014 |  | BML-QDF-CL-014 |  |  |  |

| pH of Water result | Limits | pH of sample result |  | Limits |  | Description | Remark |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5.0 – 7.0 |  |  | 3.20 – 6.50 |  | Clear and particles free solution |  |  |  |
| In – process sample Assay result |  |  |  |  |  |  |  |  |  |
| Sample quantity | Active Ingredient |  | Specification |  | Active Content (%) |  |  | Remark |  |
| 500ml | Dextrose Monohydrate B.P. |  | 95% – 105% |  |  |  |  |  |  |
| Remark: The batch is released / not released for filling. (Attach the released slip into the BMR) |  |  |  |  |  |  |  |  |  |
| Q.A. Analyst Q.C Head Sign/Date Sign /Date |  |  |  |  |  |  |  |  |  |

| FILLING LINE CLEARANCE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Time: | Clean |  | Not Clean |  |
| Area |  |  |  |  |
| Personnel |  |  |  |  |
| Equipment/Vessel | Equipment/Vessel No. | Cleaned By | Status | Inspected by QA |
| Semi-Automatic filling machine 1 | BL/FR/PE/001 |  |  |  |
| Semi-Automatic filling machine 2 | BL/FR/PE/002 |  |  |  |
| Semi-Automatic filling machine 3 | BL/FR/PE/003 |  |  |  |
| Semi-Automatic filling machine 4 | BL/FR/PE/004 |  |  |  |
| Semi-Automatic filling machine 5 | BL/FR/PE/005 |  |  |  |
| Dynamic pass box | BL/PTB/PE/001 |  |  |  |
| Present / Absent of previous product | Parameters | Specification | Result |  |
| Previous Product: | Colour | Colourless |  |  |
| Batch No.: | Identification | Must comply |  |  |
| Present Product: | pH | 5.0-7.0 |  |  |
| Batch No.: | Conductivity | NMT 1.3µs |  |  |
| Remark: | QA Officer: |  | Sign/ Date |  |

| FILLING RECORDS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Stage | FILLING INSTRUCTIONS |  | Time Started | Time Stopped | Done by | Checked by Pharmacist/Supervisor | Verified by QA |
| A. | Time : |  |  |  |  |  |  |
| 1. | Collect the appropriate product pouches & pouch size/batch size from batch coding section |  |  |  |  |  |  |
| 2. | Rinse filling machine with 75 litres of hot sterile apyrogenic water |  |  |  |  |  |  |
| 3. | Rinse filling machine with 15 litres of prepared product |  |  |  |  |  |  |
| 4. | Fill products into PVC Pouches |  |  |  |  |  |  |
| 5. | Take samples of initial filling for QC with green/masking tape tags |  |  |  |  |  |  |
| 6. | Check volume filled on each machine every 20-30 minutes and take necessary corrective action. Record volume filled in log sheet |  |  |  |  |  |  |
| 7. | Use stoppers to seal the filled bags |  |  |  |  |  |  |
| 8. | Transfer bags through the pass-box to the loading bay |  |  |  |  |  |  |
| 9. | Load filled bags onto trays/trolleys and record total quantity loaded |  |  |  |  |  |  |
| 10. | N.B. Use 0.8-, 0.45- and 0.2-micron filters |  |  |  |  |  |  |
| Production Pharmacist/Supervisor |  | Date |  | Quality Assurance Officer |  |  | Date |

| PERIODIC VOLUME CHECK |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stage | CORRELATED IN TERMS OF FILLED VOLUME |  |  |  |  |  |  |  |  |  |  |
|  | Date : Time started: |  |  |  |  |  | Filling Specification: 1000ml – 1020ml |  |  |  |  |
|  |  |  |  |  |  |  | Time Stopped: |  |  |  |  |
| - Adjust the volume to not less than the expected final volume. - Cross check using measuring cylinder at delivery point of the nozzles. - Check the volume of filled pouch by cutting with a stainless steel scissors and measure the content volume using a measuring cylinder - Check the volume every 20-30 minutes of filling operation |  |  |  |  |  |  |  |  |  |  |  |
|  | FILLED VOLUME FROM DELIVERED NOZZLES |  |  |  |  |  |  |  |  |  |  |
| TIME | 1 | 2 | 3 |  | 4 | 5 | 6 | 7 | 8 | 9 | CHECKED BY |
|  | FILLED VOLUME FROM DELIVERED NOZZLES |  |  |  |  |  |  |  |  |  |  |
| TIME | 10 | 11 | 12 |  | 13 | 14 | 15 | 16 | 17 | 18 | CHECKED BY |
|  | FILLED VOLUME FROM DELIVERED NOZZLES |  |  |  |  |  |  |  |  |  |  |
| TIME | 19 | 20 | 21 |  | 22 | 23 | 24 | 25 | 26 | 27 | CHECKED BY |
| RECONCILIATION OF FILLING: - Total no. of Pouches coded = No. of Pouches. - Actual no. of Pouches filled = No. of Pouches. - No. of Pouches rejected = No. of Pouches. - No. of Pouches unfilled = No. of Pouches. - Retention sample with BMR = No. of Pouches. - Net yield (B + E) = No. of Pouches. - % Net yield (F x 100 / A) = % Destroy rejected Pouches as per SOP No.: BML/PDF/PR/031 Done by: |  |  |  |  |  |  |  |  |  |  |  |
| Production Pharmacist/Supervisor |  |  |  | DATE |  | Q.A. Officer |  |  |  | DATE |  |

STERILIZATION RECORDS Previous Product:; B.NO:. Check List; Checked by (Production); Verified by (QA). Area Cleanliness. Check the cleanliness of the Autoclave. Ensure absence of materials from Previous product/batch.. Thermograph paper is in place. Chemical indicator availability. Stage; INSTRUCTIONS; Record; Operator; DATE. Loading; Pharmalab Autoclave; Name. Sign. Autoclave number. Machine Code. Quantity loaded. Parameters; Start time; End time; Pressure; Temperature. Pure water in. Sterilization; Recirculation. Heating. Sterilization. Post treatment. Cooling; Cooling 1. Cooling 2. Pressure Equalization. Process Ended. Total Time. Off Loading; Quantity leaked. QA samples. Total quantity offloaded. Percentage yield(%). Remark. . Signature; Engineer In charge; Production Pharm/Supervisor; QA Officer. Stage; De lama Autoclave; Record; Operator; DATE. Loading; Machine Code. Cleaned By; Name. Sign. Quantity Loaded. Sterilization; Stage; Parameter. Start time; End time; Pressure; Temp; F0. Water Loading. Water Recycling. Heating. Sterilization. Slow Cooling. Cooling. Water Drainage & Pressure Exhaust. End Cycle. Offloading; Quantity leaked. QA samples. Total quantity offloaded. Percentage yield(%). Remarks. Engineer in-charge; Production Pharmacist/Supervisor; QA Officer. Thermograph to be checked & signed by Operator, Engineering in charge & Pharmacist/Supervisor & attach to BMR..

PACKAGING RECORDS/CHECKLIST

| Time Sterilization Ended: Time Packaging starts: |  |  |
| --- | --- | --- |
| LINE CLEARANCE – Packing Operation Date: |  |  |
| Previous Product : | B.NO: |  |
| Check List | Checked by (Production) | Verified by ( QA ) |
| Ensure all the personnel are in appropriate overall. |  |  |
| Check area and equipment cleanliness (Floor, walls, ceiling, conveyor belt, viewing board leak-test machine). |  |  |
| Check the cleanliness of the Packing Line |  |  |
| Ensure absence of materials from Previous product/batch. |  |  |
| Ensure proper batch sterilized filled Pouches are brought in from offloading area |  |  |
| Ensure sterilized filled pouches are already cleared by QA before packing |  |  |
| Ensure all proper material for packing are available |  |  |
| Check the coded Cartons for details of coding and if it tallies with the batch details on the product for packing |  |  |

| Stage | Packaging Instructions |  |  |  |  |  |  |  |  | Signature |  | Date |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. | Offload sterilized product from the trolleys. QUANTITY OFFLOADED: ..............................… |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. | Check and record all leakages on sterilization QUANTITY LEAKED: ...........… |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PACKAGING INSTRUCTIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  | Ensure that all packaging materials are assembled and are in their correct quantities |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  | Check individual pouches for particles using viewing light. QUANTITY OF BAGS WITH PARTICLES: ..............................… |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  | Check individual bags for leakages. QUANTITY OF LEAKING POUCHES: .................. % OF LEAKAGES: .......................… |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  |  | Complete packing of the sterilized product Time completed |  |  |  |  |  |  |  |  |  |  |  |
| 5. |  |  | Record quantity packed for each product/pack. QUANTITY PACKED: ..................................................................... TIME TAKEN: ................................................................................ NUMBER OF STAFF: ...................................................................... PACKAGING LINE LEADER: ........................................................... |  |  |  |  |  |  |  |  |  |  |  |
| 6. |  |  | Store in quarantine store area until QA release |  |  |  |  |  |  |  |  |  |  |  |
| 7. |  |  | Transfer of finished product to store |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Quantity transferred: ........................................... |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Quantity checked by Internal auditor: ................................................. |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Production Supervisor: ......................................... |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Store officer: ....................................................... |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Quantity received by store officer: .......................................................… |  |  |  |  |  |  |  |  |  |  |  |
| PACKAGING RECONCILIATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  |  |  |  |  | Time started: |  | Time Completed: |  |  |  |  |  |  |
| Number of filled pouches received from autoclaving section |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Leakages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Deformed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Visible particles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Under filled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Over filled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bleached |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TOTAL REJECT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Retention/Stability sample |  |  |  | Quantity sampled: |  |  |  |  | Sampled by: |  |  |  |  |  |
| Percentage yield (%) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Production Chemist sign |  |  |  |  | Date |  | Quality Assurance Officer sign |  |  |  |  | Date |  |  |

| GENERAL RECONCILIATION |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Number of bags coded |  | Batch coding reject(s): |  | Batch coding actual % yield: |  |
| Number of bags filled |  | Number of bags unfilled Filling reject(s) |  | Filling actual %yield |  |
| Number of bags loaded |  | Loading reject(s) |  | Loading actual yield% |  |
| Number of bags autoclaved |  | Autoclaving reject(s) |  | Autoclaving actual yield % |  |
| Number of pouches packed |  | Packaging reject(s) |  | Packing actual yield % |  |
| QA pouch sample ( must be attached to BMR) |  |  |  |  |  |
| Total quantities sampled for both Q.A. analysis and retention. |  |  |  |  |  |
| Quantity transferred to FG Store |  |  |  |  |  |
| Standard yield(No’s) |  |  |  |  |  |
| Actual Yield (No’s) |  |  |  |  |  |
| Actual Yield (%) |  |  |  |  |  |
| Production Manager | Date |  | Released By Quality Assurance Manager |  | Date |

(Investigation should be made if the yield result does not meet specification) Comment: __________________________________________ Deviation (if any)……………………………………………………………………………………………………………..……………………. ………………………………………………………………………………………………………………………………………………………………

Declaration:

The above-mentioned batch has been manufactured in accordance with In-House & B.P/USP, Specifications following strict adherence to cGMP procedures and is duly released for sales after all production records, environmental monitoring records and Q.C. analytical reports are checked and found to be satisfactory.

CHECKLIST OF BATCH RECORD

| SN | Description |  | Document availability checked by |  |
| --- | --- | --- | --- | --- |
|  |  |  | Production | QA |
| 1 | BATCH DETAILS |  |  |  |
| 2 | DISPENSING MATERIAL LABELS (IF ANY) |  |  |  |
| 3 | APPROVED CODED POUCH FOR THE BATCH |  |  |  |
| 4 | APPROVED CODED MONO CARTON FOR THE BATCH (IF ANY) |  |  |  |
| 5 | THERMOGRAPH/CHEMICAL INDICATOR |  |  |  |
| 6 | PRODUCT RECONCILIATION |  |  |  |
| 7 | INVESTIGATION/DEVIATION REPORT (IF ANY) |  |  |  |
| 8 | DULY FILLED AND SIGNED? |  |  |  |
| QUALITY CONTROL REPORT AND MISCELLANOUS DOCUMENTS (Checked by QA) |  |  |  |  |
| 9 | BET REPORT/PYROGEN REPORT |  |  |  |
| 10 | STERILITY TEST REPORT |  |  |  |
| 11 | FINISHED PRODUCT (CERTIFICATE OF ANALYSIS |  |  |  |
| 12 | PRODUCT RELEASE SLIP |  |  |  |

| BMR CHECKED BY: | DATE: | SIGN: |
| --- | --- | --- |