- OBJECTIVE:

To provide information on how to de-fumigate the clean area and non-clean area.

- SCOPE:

To provide written guidelines for effective de-fumigation of the clean area and non-clean area.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Lab Assistant, Production Technician, Engineering personnel, QA Officer.

- ABBREVIATION AND DEFINITIONS

NIL

- PROCEDURES

DE-FUMIGATION OPERATION

- After four hours or overnight of fumigation activity, the Technical Department begins de-fumigation by switch on the Air Handling Unit.

- Remove the Aluminum foils wrapped around the open lines and machine nozzles by the production operator.

- Open the fresh air supply dampers and exhaust dampers next morning.

- Close the recirculation dampers.

- Do not enter the aseptic area before 30 minutes of starting the AHU’s.

- Clean all the areas as per respective area cleaning SOP.

- Clean all the equipment before use as per their respective SOP’s

- After attaining the required temperature and relative humidity, take line clearance and then start the activities. (As applicable).

Post-De-Fumigation Steps

- After sufficient ventilation, QA personnel should enter the area and visually inspect all surfaces for any signs of fumigant residues.

- Any visible residues found, it should be cleaned up according to the established cleaning procedure.

- Record the completion of the de-fumigation process in the log sheet, including any observations or cleaning actions taken.

- Once all checks are completed and confirmed, the area can be released for routine production or other activities.

Destruction of excess quantity of fumigating agent:

- After fumigation exercise, transfer the container of the fumigating agent to the washing area.

- Add potable water into the container of the fumigating agent  and dilute

- After dilution, drain into the wash area drain followed by flushing of drain with water.

SAFETY, HEALTH AND ENVIRONMENTAL PRECAUTIONS:

- Always wear protective coverings and hand gloves while handling fumigant.

- REFERENCE AND ENCLOSURE

ENCLOSURE 1- FUMIGATION LOGBOOK- BML-QDG-QA-014-F01

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |