- OBJECTIVE:

To lay down the procedure for conducting an investigation of Out of trend (00T) results observed during laboratory testing of samples.

- SCOPE:

This SOP is applicable to the out of trend results observed in the analysis of raw materials, packing materials, finished product, in process samples, environmental monitoring results, water results and stability samples with respect to the previous stability test points conducted.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Quality Control Officer: To notify the Executive/ Assistant Manager QC about the 00T result

Head Quality Control: To authorize for the re-analysis.

Head QC shall be responsible to ensure compliance of this SOP.

Head Quality Assurance- Review and Closing of Investigation Report.

- ABBREVIATION AND DEFINITIONS

SOP: Standard Operating Procedure

QC: Quality Control

OOT: Out of Trend

- PROCEDURES

- Out of trend result shall be determined on the basis of the data of previous batches or previous analytical data or aberrant data observed during analysis. The result may be within the specification but not in the trend.

- While performing the analysis, if any OOT or aberrant data is observed, then the analyst shall immediately inform the QC Head of the section / Assistant Manager QC shall conduct the preliminary investigation along with the analyst of the 00T observation as per Section - I of Enclosure 1 (00T Investigation Form).

- QC analyst shall store the sample and standard preparations and any other preparations used during the analysis until the investigation is completed.

- Make the necessary entries in the OOT log (Enclsoure-2) by assigning the OOT No. as follows: OT/XX/ YYY

Where,

OT: Out of Trend

XX: Last two digits of the current year (e.g. for 2019 mention 19

YYY: Sequential number (e.g. 001,002 ......)

- During the above investigation, if the cause of OOT result is identified as laboratory error and the Executive QC of the section / Assistant Manager QC and Head Quality Control conclude on it and repeat analysis shall be carried out and reported.

- The same analyst shall repeat the test with a fresh sample and fresh standard preparation (if required) with due care to omit the errors identified.

- In case if the cause of OOT result is not identified as laboratory error then Head QC shall recommend for re-analysis to confirm the OOT results.

- If the results of repeat analysis are similar to a previous analysis the OOT results are valid and the same shall be informed to the concerned departments.

- The analyst shall record the re-analysis results in the worksheet.

- If the reanalysis results are within the trend and observations are not aberrant, then the initial analysis shall be investigated for the cause of OOT and report the repeat analysis results.

- If the cause of OOT results is not identified, and if the results of re-analysis are same as the previous analysis then the results obtained during the first analysis shall be reported.

- If the test result is out of trend, it shall be concluded that the material/product is not meeting the trends and the final decision for acceptance of material/product shall be taken by Head Quality.

- In the case of raw and packing material: Head Quality control shall review the OOT results and forward the same to QA and Supplier for investigation and prevent the recurrence of such incident in the future supplies.

- In case of Finished Products: Head - Quality Control shall review the OOT results and forward the same to QA and Production for information and investigation.

- A final decision will be taken by the head Quality based on the investigation report.

- In case of Stability studies: OOT investigations to be carried out and if required analysis can be performed using the samples of the previous stability test point. A final decision will be taken by the head Quality based on the investigation report in coordination with Production and R&D (If applicable).

- If the product belongs to another customer where OOT is observed the same shall be informed to the product owner.

- REFERENCE AND ENCLOSURE

- Water Quality Manual

- ISO 14644-1 guidelines

- BP (British Pharmacopoeia)

- Annual Product Review (ARV)

- Enclosure 1 –00T Investigation Form

- Enclosure 2 - Out of Trend (OOT) log

7.0 TRAINING INVOLVEMENT

| Quality Control Head | Quality Control Analyst |
| --- | --- |
| QA Doc. Unit staff | QA Officer |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |