| Raw Material Name: Dextrose Monohydrate | Batch No: | Pharmacopeia Ref: BP 2023/IN-HOUSE |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RMA002 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No. | Test | Specifications | Results |
| --- | --- | --- | --- |
|  | Description | A White or almost white, crystalline powder |  |
|  | Solubility | freely soluble in water, very slightly soluble in ethanol (96%). |  |
|  | Identification | - Specific Optical Rotation: +52.5 to +53.3 (anhydrous substance) - HPLC: The principal peak in the chromatogram obtained with the test solution is similar to the principal peak in the chromatogram obtained with reference solution - Water: 7.5% to 9.5% |  |
|  | Appearance of solution | The solution is clear and not more intensely coloured than reference solution |  |
|  | Conductivity | NMT 20 µS⋅cm-1 |  |
|  | ### Related substances | Sum of impurities A and B: NMT 0.4% Impurity C: MNT 0.2% Impurity D: NMT 0.15% Unspecified impurities: NMT 0.1% Total: NMT 0.5% Disregard limit: NMT 0.05% |  |
|  | ### Dextrin | The substance dissolves completely. |  |
|  | ### Soluble starch, sulfite | NMT 15ppm |  |
|  | ### Assay | 97.5 per cent to 102.0 per cent (anhydrous substance) |  |
|  | Total Microbial Count Total yeast &moulds Pathogens | ≤10CFU/ml ≤10CFU/ml Nil. Must be Absent |  |
|  | Bacteria Endotoxin | ≤1.15OC (3 Rabbits sum response) ≤2.5EU/mL(LAL Test) |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: