| Raw Material Name: Potassium Chloride | Batch No: | Pharmacopeia Ref: BP 2023/IN-HOUSE |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RMA003 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No. | Test | Specifications | Results |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder or colourless crystals. |  |
|  | Solubility | Freely soluble in water, practically insoluble in anhydrous ethanol. |  |
|  | Identification | - It gives reaction of chlorides - It gives reaction of Potassium |  |
|  | Acidity/Alkalinity | Not more than 0.5mL of 0.01 M hydrochloric acid or 0.01 M sodium hydroxide is required to change the colour of the indicator. |  |
|  | Bromides | NMT 0.1% |  |
|  | Sulphates | 300ppm |  |
|  | Iodides | After 5 min, examine in daylight. The substance shows no blue colour. |  |
|  | Aluminium | NMT 1.0 ppm |  |
|  | Magnesium and alkaline-earth metals | Maximum 200ppm |  |
|  | Sodium | NMT 0.1% |  |
|  | Sulphate | ≤300ppm |  |
|  | Loss on Drying | Maximum 1.0 per cent, determined on 1.000 g by drying in an oven at 105 °C for 2 h. |  |
|  | Assay | 99.0 per cent to 101.0 per cent of KCl (dried substance). |  |
|  | Total Microbial Count Total yeast &moulds Pathogens | ≤10CFU/ml ≤10CFU/ml Nil. Must be Absent |  |
|  | Bacteria Endotoxin | ≤1.15OC (3 Rabbits sum response) ≤2.5EU/ml(LAL Test) |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: