| Raw Material Name: Sodium Chloride | Batch No: | Pharmacopeia Ref: BP 2023/IN-HOUSE |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RMA001 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No. | Test | Specifications | Results |
| --- | --- | --- | --- |
|  | Description | A White or almost white, crystalline powder or colorless crystals or white or almost white pearls |  |
|  | Solubility | Freely soluble in water, practically insoluble in anhydrous ethanol. |  |
|  | Identification | - It gives reaction of chlorides - It gives reaction of Sodium |  |
|  | Appearance of solution | Solution S is clear and colourless |  |
|  | Acidity/Alkalinity | NMT 0.5 mL of 0.01 M hydrochloric acid or 0.01 M sodium hydroxide is required to change the colour of the indicator. |  |
|  | Bromides | ≤100ppm |  |
|  | Ferrocyanides | No blue colour develops within 10 min |  |
|  | Iodides | After 5 min, examine in daylight. The mixture shows no blue colour. |  |
|  | Nitrites | The absorbance is not greater than 0.01 at 354 nm. |  |
|  | Phosphates | NMT 25ppm |  |
|  | Sulphate | ≤200ppm |  |
|  | Aluminum | NMT 0.2ppm |  |
|  | Arsenic | ≤1 ppm |  |
|  | Barium | Opalescence in the solution is not more intense than that in a mixture of 7mL of distilled water R and 5 mL of solution S. |  |
|  | Iron | ≤2ppm |  |
|  | Magnesium and Akali -earth metal | NMT 100ppm |  |
|  | Potassium | 500ppm |  |
|  | Loss on Drying | Maximum 0.5 per cent, determined on 1.000 g by drying in an oven at 105 °C for 2 h. |  |
|  | Assay | 99.0 – 100.5% |  |
|  | Total Microbial Count Total yeast & moulds Pathogens | ≤10CFU/ml ≤10CFU/ml Nil. Must be Absent |  |
|  | Bacteria Endotoxin | ≤1.15OC (3 Rabbits sum response) ≤2.5EU/mL(LAL Test) |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: