RAW MATERIAL NAME: POVIDONE

SOP Ref. No.: BML-QDG-QA-105                                                            	 EFFECTIVE DATE: JAN., 2024

| S/No. | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATIONS REFERENCES |
| --- | --- | --- | --- |
| 1. | Description | White yellowish-white hygroscopic powder or flakes | BP 2023 |
| 2. | Solubility | Freely soluble in water, in ethanol, in methanol, very slightly soluble in acetone | BP 2023 |
| 3. | Identification | - A red colour is produced with 0.05M I2 solution - The substance dissolves in water | BP 2023 |
| 4. | Clarity of solution | The solution is clear and not more coloured than reference solution | BP 2023 |
| 5. | Impurity A | Must comply | BP 2023 |
| 6. | Total impurities | Must comply | BP 2023 |
| 7. | Iron (Heavy Metal) | ≤ 10ppm pb | BP 2023 |
| 8. | Residue on ignition | ≤ 0.1% | BP 2023 |
| 9. | Loss on drying | NMT 5% | BP 2023 |
| 10. | Sulphated Ash | NMT 1% | BP 2023 |
| 11. | Assay | 5.5 – 8.5/1.215 -1.275 | BP 2023 |
| 12. | Total Microbial count Total Yeast & Moulds Count Pathogens | ≤10CFU/ml ≤10CFU/ml Must be Absent | BP 2023 |
| 13. | Bacteria Endotoxin | ≤1.15 oC ( Rabbit Test) ≤0.25EU/ml (LAL Test) | BP 2023 |

EU/ml = Endotoxin units per millilitre, ml = milliliter, LAL= Limulus Amoebacyte Lysate, NMT= Not More Than, CFU/ml= Colony Forming Unit per Millilitre