| Raw Material Name: Ascorbic Acid | Batch No: | Pharmacopeia Ref: BP 2024 |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RMA018 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Appearance |  | White or almost white, crystalline powder or colourless crystals, becoming discoloured on exposure to air and moisture |  |
| 2. | Solubility |  | Very soluble in water, sparingly soluble in ethanol (96%) |  |
| 3. | Melting point |  | About 190oC, with decomposition |  |
| 4. | Identification |  | - UV absorption spectrophotometry: specific absorbance at the absorption maximum, 243nm: 545 to 585 - IR absorption spectrum: similar to ascorbic acid CRS - pH: 2.1 to 2.6 A grey precipitate is formed on addition of silver nitrate. |  |
| 5. | Appearance of solution |  | Solution S is not more intensely coloured than reference solution |  |
| 6. | Specific optical rotation |  | +20.5 to +21.5 |  |
| 7. | Impurity E |  | NMT 0.2% |  |
| 8. | Assay |  | 99.0 – 100.5% |  |
| 9. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: