| Raw Material Name: Ibuprofen | Batch No: | Pharmacopeia Ref: BP 2024 |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RMA015 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Appearance |  | White or almost white, crystalline powder or colourless crystals. |  |
| 2. | Solubility |  | Practically insoluble in water, freely soluble in acetone, in methanol and in methylene chloride. It dissolves in dilute solutions of alkali hydroxides and carbonates. |  |
| 3. | Identification |  | - Melting point: 75 °C to 78 °C. - UV-Vis spectrophotometry: absorption maxima at 264 and 27nm with a shoulder at 258nm |  |
| 4. | Appearance of solution |  | Solution is clear and colourless |  |
| 5. | Optical rotation |  | -0.05 to +0.05o |  |
| 6. | Heavy metals |  | NMT 10ppm |  |
| 7. | Loss on drying |  | NMT 0.5% |  |
| 8. | Sulphated ash |  | NMT 0.1% |  |
| 9. | Assay |  | 98.5-101.0% |  |
| 10. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: