| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Appearance |  | Clear, colourless, syrupy liquid, miscible with water |  |
| 2. | Solubility |  | Miscible with water and with ethanol (96 per cent). |  |
| 3. | Identification |  | - Chromatogram obtained is similar in retention time to the principal peak in that of the reference solution. - Angle of rotation of treated sample is + 1.5° to + 3.5° - It is a clear, syrupy liquid at 25 °C. |  |
| 4. | Conductivity |  | NMT 10 µS⋅cm–1 for undiluted liquid sorbitol |  |
| 5. | Reducing sugars |  | NMT 0.2% calculated as glucose equivalent |  |
| 6. | Reducing sugars after hydrolysis |  | NMT 9.3% calculated as glucose equivalent |  |
| 7. | Lead |  | NMT 0.5ppm |  |
| 8. | Nickel |  | NMT 1ppm |  |
| 9. | Water |  | 28.0% to 32.0% on 1.00g |  |
| 10. | Assay |  | 68.0% to 72.0% anhydrous substance |  |
|  |  |  | 72.0% -92.0% D-sorbitol |  |
| 11. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

| Raw Material Name: Sorbitol | Batch No: | Pharmacopeia Ref: BP 2024 |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RME001 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: