| Raw Material Name: Sulphamethoxazole | Batch No: | Pharmacopeia Ref: BP 2024 |
| --- | --- | --- |
| Manufacturer: | Supplier: | Logbook/Revision Number |
| MF. Date: | EX. Date: | Logbook page No.: |
| Sampled By: | Qty Received: | Logbook Volume: |
| Product Code: RMA014 | No. of containers: | Analysed by: |
| QC. AR. No: | Test Completed Date: | SOP Ref. No: BML-QDG-QA-105 |

| S/No: | Test |  | Specification | Result |
| --- | --- | --- | --- | --- |
| 1. | Description |  | White or yellowish-white, crystalline powder or crystals. |  |
| 2. | Solubility |  | Very slightly soluble in water, soluble in acetone, sparingly soluble in ethanol (96 per cent). It dissolves in dilute solutions of alkali hydroxides and in dilute mineral acids. |  |
| 3. | Melting Point |  | About 210 °C. |  |
| 4. | Identification |  | - Infrared absorption spectrophotometry. Comparison sulfamethizole CRS. - Thin-layer chromatography. Results A The principal spot in the chromatogram obtained with the test solution is similar in position and size to the principal spot in the chromatogram obtained with the reference solution. Detection B Spray with a 40 g/L solution of copper acetate R and examine in daylight. Results B The principal spot in the chromatogram obtained with the test solution is similar in colour to the principal spot in the chromatogram obtained with the reference solution. |  |
| 5. | Appearance of Solution |  | The solution is not more intensely coloured than reference solution Y5, BY5 or GY5. |  |
| 6. | Acidity/Alkalinity |  | Not more than 0.5 mL of 0.1 M sodium hydroxide is required to change the colour of the indicator. |  |
| 8. | Loss on Drying |  | Maximum 0.5 per cent |  |
| 9. | Sulphated Ash |  | Maximum 0.1 per cent |  |
| 10. | Assay |  | 99.0 per cent to 101.0 per cent (dried substance). |  |
| 11. | Microbial Contamination | Total Microbial count | NMT 1000cfu/g |  |
|  |  | Total Yeast & Moulds Count | NMT 100cfu/g |  |
|  |  | Pathogens | Must be Absent |  |

CHECKER’s VERDICT (box option):  RELEASED   / FAILED / HOLD   / REPEAT

Remark: