| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, anhydrous formic acid, methylene chloride and in ethanol (96 per cent). |
|  | IDENTIFICATION | Infrared absorption spectrophotometry |
|  | APPEARANCE OF SOLUTION | ### Solution S Dissolve 0.10 g in a mixture of 1 volume of anhydrous formic acid R and 9 volumes of methylene chloride R and dilute to 10 mL with the same mixture of solvents. |
|  | RELATED SUBSTANCES | Liquid chromatography. Test solution Dissolve 25.0 mg of the substance to be examined in 5 mL of methanol R containing 1 per cent V/V of sulfuric acid R and dilute to 50.0 mL with the mobile phase. Reference solution (a) Dissolve 10.0 mg of the substance to be examined in 10 mL of methanol R containing 1 per cent V/V of sulfuric acid R and dilute to 100.0 mL with the mobile phase. Dilute 0.5 mL of this solution to 20.0 mL with the mobile phase. Reference solution (b) Dissolve 50.0 mg of the substance to be examined and 50 mg of oxibendazole CRS in 5 mL of methanol R containing 1 per cent V/V of sulfuric acid R and dilute to 100.0 mL with the mobile phase. Column: — size: l = 0.25 m, Ø = 4.6 mm; — stationary phase: spherical end-capped octadecylsilyl silica gel for chromatography R (5 µm) with a pore size of 10 nm and a carbon loading of 19 per cent. Mobile phase Mix 300 volumes of a 1.67 g/L solution of ammonium dihydrogen phosphate R and 700 volumes of methanol R. Flow rate 0.7 mL/min. Detection Spectrophotometer at 254 nm. Injection 20 µL. Run time 1.5 times the retention time of albendazole. Relative retention With reference to albendazole: impurity D = about 0.40; impurities B and C = about 0.43; impurity E = about 0.47; impurity F = about 0.57; impurity A = about 0.80. System suitability: reference solution (b): — resolution: minimum 3.0 between the peaks due to albendazole and oxibendazole. |
|  | LOSS ON DRYING | Determined the Loss on drying on 1.000 g by drying in an oven at 105 °C for 4hours |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample at 8000C |
|  | ASSAY | Dissolve 0.250 g in 3 mL of anhydrous formic acid R and add 40 mL of anhydrous acetic acid R. Titrate with 0.1 M perchloric acid, determining the end-point potentiometrically (2.2.20). 1 mL of 0.1 M perchloric acid is equivalent to 26.53 mg of C12H15N3O2S. Note: In order to avoid overheating in the reaction medium, mix thoroughly throughout and stop the titration immediately after the end-point has been reached. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |