| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe colour and other characteristics of the material. |
|  | SOLUBILITY | Determine solubility of material in water, ethanol (96 percent), hexane and in methylene chloride. |
|  | IDENTIFICATION | First identification B Second identification A, C, D A. Ultraviolet and visible absorption spetrophotometry Dissolve 0.1g in ethanol (96 %) R and dilute to 100mL with the same solvent. Measure absorbance of the solution between 230 nm and 300 nm and observe wavelengths of maximum absorption. B. Infrared absorption spectrophometry, comparing with the spectrum obtained with aspartame CRS. Examine the substances prepared as discs. C. Thin layer chromatography. Test solution Dissolve 15 mg of the substance to be examined in 2.5 mL of water R and dilute to 10 mL with acetic acid R. Reference solution Dissolve 15 mg of aspartame CRS in 2.5 mL of water R and dilute to 10 mL with acetic acid R. Plate:TLC silica gel G plate R Mobile phase water R, anhydrous formic acid , methanol R, methylene chloride R (2:4:30:64 V/V/V/V) Application: 20 µL Development ; Over a path of 15 cm. Drying: In air. Detection: Spray with ninhydrin solution R and heat at 100 0C to 105 0C for 15 minutes. D. Dissolve about 20 mg in 5 mL of methanol R and add 1 mL of alkaline hydroxylamine solution R1. Heat on a water bath for 15 minutes. Allow to cool and adjust to about pH 2 with dilute hydrochloric acid R. Add 0.1 mL of ferric chloride solution R1. Note any coloured solution formed. |
|  | APPEARANCE OF SOLUTION | SOLUTION S: Dissolve 0.8 g in carbon dioxide – free water and dilute to 100 mL with the same solvent. Observe and record the clarity and colour intensity of solution S in comparison with reference solution GY6. |
|  | CONDUCTIVITY | Dissolve 0.80 g in carbon dioxide – free water R prepared from distilled water R and dilute to 100.0 mL with the same solvent. Measure the conductivity of the solution (C1) and that of the water used for preparing the solution (C2). The readings must be stable within 1% over a period of 30 seconds. Calculate the conductivity of the solution of the substance to be examined from the expression: C1 – 0.992C2 |
|  | SPECIFIC OPTICAL ROTATION | Dissolve 2.00 g in a 690 g/L solution of anhydrous formic acid R and dilute to 50.0 mL with the same solution. Measure the specific optical rotation within 30 minutes of preparation of the solution and calculate with reference to the dried substance. |
|  | RELATED SUBSTANCES | Liquid chromatography Test solution Dissolve 0.60 g of the substance to be examined in a mixture of 1.5 volumes of glacial acetic acid R and 98.5 volumes of water R and dilute to 100.0 mL with the same mixture of solvents. Reference solution (a) Dissolve 4.5 mg of aspartame impurity A CRS in a mixture of 1.5 volumes of glacial acetic acid R and 98.5 volumes of water R and dilute to 50.0 mL with the same mixture of solvents. Reference solution (b) Dissolve 30.0 mg of phenylalamine R (impurity C) in a mixture of 15 volumes of glacial acetic acid R and 85 volumes of water R and dilute to 100.0 mL with the same mixture of solvents. Dilute 1.0 mL of this solution to 10.0 mL with water R. Reference solution (c) Dilute 5.0 mL of the test solution to 10. 0 mL with water R. Dilute 3.0 mL of this solution to 100.0 mL with water R. Reference solution (d) Dissolve 30.0 mg of L-aspartyl-L-phenylalanine R (impurity B) in a mixture of 15 volumes of glacial acetic acid R and 85 volumes of water R and dilute to 100.0 mL with the same mixture of solvents. Dilute 1.0 mL of this solution with 1.0 mL of reference solution (b). Column: ---- size: l = 0.25 m, Ø = 4.0 mm ---- stationary phase: octadecylsilyl silica gel for chromatography R (5 – 10 µm). Mobile phase Mix 10 volumes of acetonitrile R and 90 volumes of a 6.8 g/L solution of potassium dihydrogen phosphate R, previously adjusted to pH 3.7 with phosphoric acid R. Flow rate 1 mL/min Detection Spectrophotometer at 220 nm Injection 20 µL Run time Twicethe retention time of aspartame. System suitability: Reference solution (d): ----- resolution: minimum 3.5 between the peaks due to impurities B and C. |
|  | LOSS ON DRYING | Determine the loss on drying as in General Method No. 002 using 1.000 g @ 105 0C. |
|  | SULPHATED ASH | Determine the sulphated ash on 1.0 g sample at 8000C. |
|  | ASSAY | Dissolve 0.250 g in 1.5 mL of anhydrous formic acid R and 60 mL of anhydrous acetic acid R. Titrate immediately with 0.1M perchloric acid, determining the end point potentiometrically. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |