| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine it solubility in water, in ethanol (96 per cent) and insoluble in heptane. |
|  | IDENTIFICATION | First identification: A, B, D. Second identification: C, D. A. Determine the Specific optical rotation. Dissolve 2.50 g in carbon dioxide-free water R and dilute to 50.0 mL with the same solvent. B. Infrared absorption spectrophotometry. Comparisoncalcium pantothenate CRS. C. Thin-layer chromatography. Test solution Dissolve 10 mg of the substance to be examined in a mixture of 0.25 mL of water R and 4 mL of methanol R. Reference solution Dissolve 10 mg of calcium pantothenate CRS in a mixture of 0.25 mL of water R and 4 mL of methanol R. PlateTLC silica gel plate R. Mobile phaseglacial acetic acid R, water R, 2-propanol R (5:15:80 V/V/V). Application 5 µL. Development Over 4/5 of the plate. Drying In air. Detection Heat at 120 °C for 20 min; treat the warm plate with a 3 g/L solution of ninhydrin R in a mixture of 3 volumes of glacial acetic acid R and 100 volumes of anhydrous ethanol R; allow to dry and heat again at 120 °C for a few minutes; examine in daylight. D. It gives reaction (a) of calcium. To 0.2 mL of a neutral solution containing a quantity of the substance to be examined equivalent to about 0.2 mg of calcium (Ca2+) per millilitre or to 0.2 mL of the prescribed solution add 0.5 mL of a 2 g/L solution of glyoxalhydroxyanil R in ethanol (96 per cent) R, 0.2 mL of dilute sodium hydroxide solution R and 0.2 mL of sodium carbonate solution R. Shake with 1 mL to 2 mL of chloroform R and add 1 mL to 2 mL of water R. The chloroform layer is coloured red. |
|  | APPEARANCE OF SOULTION | ### Solution S Dissolve 2.50 g in carbon dioxide-free water R and dilute to 50.0 mL with the same solvent. Observe the clarity of the solution |
|  | pH | Determine the PH of solution S |
|  | SPECIFIC OPTICAL ROTATION | Determine the Specific Optical Rotation of solution S. |
|  | IMPURITIES A AND OTHER AMINOCARBOXYLIC ACID IMPURITIES | Dissolve 8.000 g in 40 mL of water R and dilute to 100 mL with the same solvent. Add 25 mL of formaldehyde solution R. Titrate with 0.1 M sodium hydroxide, determining the end-point potentiometrically (2.2.20). 1 mL of 0.1 M sodium hydroxide is equivalent to 8.91 mg of C3H7NO2.Liquid chromatography. |
|  | RELATED SUBSTANCES | Test solution Dissolve 0.600 g of the substance to be examined in water R and dilute to 100.0 mL with the same solvent. Reference solution (a) Dissolve 30.0 mg of calcium pantothenate CRS in water R and dilute to 50.0 mL with the same solvent. Reference solution (b) Dilute 1.0 mL of the test solution to 100.0 mL with water R. Dilute 1.0 mL of this solution to 10.0 mL with water R. Reference solution (c) Dissolve 3.0 mg of pantolactone CRS (impurity C) in 5.0 mL of water R. Mix 1.0 mL of the solution and 1.0 mL of reference solution (a) and dilute to 100.0 mL with the same solvent. Reference solution (d) Dissolve 3 mg of pantothenate for peak identification CRS (containing impurities B, E and H) in 0.5 mL of water R. Column: — size: l = 0.15 m, Ø = 3.0 mm; — stationary phase: octadecylsilyl silica gel for chromatography R (3.5 µm); — temperature: 35 °C. Mobile phase: — mobile phase A: mix 1 volume of acetonitrile R1 and 99 volumes of a 1.56 g/L solution of sodium dihydrogen phosphate R previously adjusted to pH 2.5 with phosphoric acid R; — mobile phase B: acetonitrile R1; Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 6; 100; 0. 6 - 21; 100 → 50; 0 → 50. 21 - 30; 50; 50. Flow rate 1.0 mL/min. Detection Spectrophotometer at 200 nm. Injection 5 µL of the test solution and reference solutions (b), (c) and (d). Identification of impurities Use the chromatogram obtained with reference solution (c) to identify the peak due to impurity C; use the chromatogram supplied with pantothenate for peak identification CRS and the chromatogram obtained with reference solution (d) to identify the peaks due to impurities B, E and H. Relative retention With reference to calcium pantothenate (retention time = about 4 min): impurity B = about 0.5; impurity C = about 0.8; impurity E = about 1.7; impurity H = about 2.3. System suitability Reference solution (c): — resolution: minimum 3.0 between the peaks due to impurity C and calcium pantothenate; — signal-to-noise ratio: minimum 10 for the peak due to impurity C. Calculation of percentage contents: — for impurities B and C, use the concentration of impurity C in reference solution (c); — for impurities other than B and C, use the concentration of calcium pantothenate in reference solution (b). |
|  | CHLORIDE | Dilute 5 mL of solution S to 15 mL with water R. Subject to the limit test for chlorides. 15 mL of the prescribed solution add 1 mL of dilute nitric acid R and pour the mixture as a single addition into a test-tube containing 1 mL of silver nitrate solution R2. Prepare a standard in the same manner using 10 mL of chloride standard solution (5 ppm Cl) R and 5 mL of water R. Examine the tubes laterally against a black background. After standing for 5 min protected from light, any opalescence in the test solution is not more intense than that in the standard. |
|  | LOSS ON DRYING | Determined on 1.000 g by drying in an oven at 105 °C. |
|  | ASSAY | Dissolve 0.180 g in 50 mL of anhydrous acetic acid R. Titrate with 0.1 M perchloric acid, determining the end-point potentiometrically (2.2.20). 1 mL of 0.1 M perchloric acid is equivalent to 23.83 mg of C18H32CaN2O10. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |