MATERIAL NAME: CHLOROQUINE PHOSPHATE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | A white or almost white, crystalline powder, hygroscopic. | BP 2023 |
|  | Solubility | Freely soluble in water, very slightly soluble in ethanol (96 per cent) and in methanol. | BP 2023 |
|  | Identification | A. shows absorption maxima at 220 nm, 235 nm, 256 nm, 329 nm and 342 nm. The specific absorbances at the maxima are respectively 600 to 660, 350 to 390, 300 to 330, 325 to 355 and 360 to 390. B. Infrared absorption spectrophotometry Comparison chloroquine sulfate CRS. C. melts at 206-209 °C D. It gives reaction (b) of phosphates | BP 2023 |
|  | Appearance of Solution | ### Solution S ### Solution S and a fivefold dilution of solution S are clear. Solution S is not more intensely coloured than reference solution BY6 | BP 2023 |
|  | pH | The pH of solution S is 3.8 to 4.3. | BP 2023 |
|  | Related Substances | — impurity A: not more than the area of the principal peak in the chromatogram obtained with reference solution (a) (0.5 per cent), — any other impurity: not more than 0.6 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.3 per cent), — total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution (a) (1.0 per cent), — disregard limit: 0.1 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.05 per cent). | BP 2023 |
|  | Melting Point | One of which melts at about 195 °C and the other at about 218 °C. | BP 2023 |
|  | Loss on Drying | Maximum 2.0 per cent, | BP 2023 |
|  | Assay | 98.5 % to 101.0 % (dried substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram