| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, acetone and in ethanol (96 per cent) |
|  | IDENTIFICATION | - Ultraviolet and visible absorption spectrophotometry Test solution Dissolve 2.5 mg in water R and dilute to 100.0 mL with the same solvent. Spectral range 260-610 nm. Absorption maxima 278 nm, 361 nm and 547-559 nm. B. Examine the chromatograms obtained in the test for related substances. |
|  | RELATED SUBSTANCES | Liquid chromatography: Use the normalisation procedure. Store the solutions at 2-8 °C, protected from light, and use them within 24 h. Test solution Dissolve 25.0 mg of the substance to be examined in water R and dilute to 50.0 mL with the same solvent. Reference solution (a) Dilute 1.0 mL of the test solution to 100.0 mL with water R. Dilute 1.0 mL of this solution to 10.0 mL with water R. Reference solution (b) Dissolve 5 mg of cyanocobalamin for system suitability CRS (containing impurities A, C, E and F) in water R and dilute to 10 mL with the same solvent. Reference solution (c) Dissolve 5.0 mg of cyanocobalamin CRS in water R and dilute to 10.0 mL with the same solvent. Reference solution (d) Dissolve 5 mg of cyanocobalamin for peak identification CRS (containing impurities B and D) in water R and dilute to 10 mL with the same solvent. Column: — size: l = 0.15 m, Ø = 2.1 mm; — stationary phase: end-capped ethylene-bridged octylsilyl silica gel for chromatography (hybrid material) R (1.7 µm); — temperature: 60 °C. Mobile phase: — mobile phase A: 1.0 g/L solution of ammonium formate R adjusted to pH 3.5 with anhydrous formic acid R; — mobile phase B: methanol R; Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 1; 90; 10. 1 - 16; 90 → 80; 10 → 20. 16 - 23; 80 → 60; 20 → 40. Flow rate 0.4 mL/min. Detection Spectrophotometer at 361 nm. Injection 3 µL. Identification of impurities Use the chromatogram supplied with cyanocobalamin for system suitability CRS and the chromatogram obtained with reference solution (b) to identify the peaks due to impurities A, C, E and F; use the chromatogram supplied with cyanocobalamin for peak identification CRS and the chromatogram obtained with reference solution (d) to identify the peaks due to impurities B and D. Relative retention With reference to cyanocobalamin (retention time = about 10 min): impurity F = about 1.06; impurity D = about 1.18; impurity C = about 1.23; impurity A = about 1.26; impurity E = about 1.33; impurity B = about 1.45. System suitability Reference solution (b): — peak-to-valley ratio: minimum 4.5, where Hp = height above the baseline of the peak due to impurity A and Hv = height above the baseline of the lowest point of the curve separating this peak from the peak due to impurity C; minimum 2.5, where Hp = height above the baseline of the peak due to impurity F and Hv = height above the baseline of the lowest point of the curve separating this peak from the peak due to cyanocobalamin. |
|  | LOSS ON DRYING | Determined the Loss on drying on 0.400 g by drying in vacuo at 105 °C for 2 h. |
|  | ASSAY | Dissolve 0.100 g in water R and dilute to 500.0 mL with the same solvent. Dilute 25.0 mL of the solution to 200.0 mL with water R. Measure the absorbance at the absorption maximum at 361 nm. Calculate the content of C63H88CoN14O14P taking the specific absorbance to be 207. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |