| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | First identification: C, D. Second identification: A, B, D. - Determine the Melting point - Ultraviolet and visible absorption spectrophotometry Test solution Dissolve 50 mg in ethanol (96 per cent) R and dilute to 100.0 mL with the same solvent. Spectral range 230-350 nm. Absorption maxima At 253 nm, 258 nm and 264 nm. Absorbance ratios: — A258/A253 = 1.1 to 1.3; — A258/A264 = 1.2 to 1.4. - Infrared absorption spectrophotometry Comparison diphenhydramine hydrochloride CRS. - Subject to reaction (a) of chlorides. Dissolve in 2 mL of water R a quantity of the substance to be examined equivalent to about 2 mg of chloride (Cl–) or use 2 mL of the prescribed solution. Acidify with dilute nitric acid R and add 0.4 mL of silver nitrate solution R1. Shake and allow to stand. A curdled, white precipitate is formed. Centrifuge and wash the precipitate with three quantities, each of 1 mL, of water R. Carry out this operation rapidly in subdued light, disregarding the fact that the supernatant solution may not become perfectly clear. Suspend the precipitate in 2 mL of water R and add 1.5 mL of ammonia R. The precipitate dissolves easily with the possible exception of a few large particles which dissolve slowly. |
|  | APPEARANCE OF SOULTION | Solution S Dissolve 1.0 g in carbon dioxide-free water R and dilute to 20 mL with the same solvent. Observe the clarity of Solution S and a five fold dilution of solution S. Compare it colour intensity with reference solution BY6. |
|  | ACIDITY/ALKALINITY | To 10 mL of solution S add 0.15 mL of methyl red solution R and 0.25 mL of 0.01 M hydrochloric acid. The solution is pink. Titrate the solution with 0.01 M sodium hydroxide until the colour changes to yellow. |
|  | REALTED SUBSTANCES | Liquid chromatography. Test solution Dissolve 70 mg of the substance to be examined in the mobile phase and dilute to 20.0 mL with the mobile phase. Dilute 2.0 mL of the solution to 10.0 mL with the mobile phase. Reference solution (a) Dilute 1.0 mL of the test solution to 10.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 20.0 mL with the mobile phase. Reference solution (b) Dissolve 5 mg of diphenhydramine impurity A CRS and 5 mg of diphenylmethanol R in the mobile phase and dilute to 10.0 mL with the mobile phase. To 2.0 mL of this solution add 1.5 mL of the test solution and dilute to 10.0 mL with the mobile phase. Column: — size: l = 0.25 m, Ø = 4.6 mm, — stationary phase: base-deactivated octylsilyl silica gel for chromatography R (5 µm). Mobile phase Mix 35 volumes of acetonitrile R and 65 volumes of a 5.4 g/L solution of potassium dihydrogen phosphate R adjusted to pH 3.0 using phosphoric acid R. Flow rate 1.2 mL/min. Detection Spectrophotometer at 220 nm. Injection 10 µL. Run time 7 times the retention time of diphenhydramine. Relative retention With reference to diphenhydramine (retention time = about 6 min): impurity A = about 0.9; impurity B = about 1.5; impurity C = about 1.8; impurity D = about 2.6; impurity E = about 5.1. System suitability Reference solution (b): — resolution: minimum 2.0 between the peaks due to diphenhydramine and to impurity A. Limits: — correction factor: for the calculation of content, multiply the peak area of impurity D by 0.7, — impurity A: not more than the area of the principal peak in the chromatogram obtained with reference solution (a) (0.5 per cent), — any other impurity: not more than 0.6 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.3 per cent), — total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution (a) (1.0 per cent), — disregard limit: 0.1 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.05 per cent). |
|  | LOSS ON DRYING | Determined the Loss on drying on 1.000 g by drying in an oven at 105 °C. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample.at 8000C |
|  | ASSAY | Dissolve 0.250 g in 50 mL of ethanol (96 per cent) R and add 5.0 mL of 0.01 M hydrochloric acid. Carry out a potentiometric titration, using 0.1 M sodium hydroxide. Read the volume added between the 2 points of inflexion. 1 mL of 0.1 M sodium hydroxide is equivalent to 29.18 mg of C17H22ClNO. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |