MATERIAL NAME: ETHANOL 96%

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | Colourless, clear, volatile, flammable liquid, hygroscopic. | BP 2023 |
|  | Solubility | Miscible with water and with methylene chloride. It burns with a blue, smokeless flame. Boiling point about 78 0C. | BP 2023 |
|  | Identification | - Relative Density: 0.805 – 0.812 - IR spectrophotometry, comparing with Ph. Eur. Reference spectrum ethanol (96%). - After a few minutes, an intense blue colour appears on the paper and becomes pale after 10 to 15 minutes. - A yellow precipitate is formed within 30 minutes. | BP 2023 |
|  | Appearance | It is clear and colourless when compared with water. After standing for 5 minutes the dilution remains clear when compared with water. | BP 2023 |
|  | Acidity or Alkalinity | On adding phenolphthalein solution, the solution is colourless but after the addition of sodium hydroxide the solution becomes pink. | BP 2023 |
|  | Relative density | 0.805 – 0.812 | BP 2023 |
|  | Absorbance | Maximum 0.40 at 240 nm, 0.30 between 250 nm and 260 nm and 0.10 between 270 nm and 340 nm. The absorption curve is smooth between when examined between 235 nm and 340 nm. | BP 2023 |
|  | Volatile impurities | -- total of other impurities in the chromatogram obtained with test solution (b): not more than the area of the peak due to 4-methylpentan-2-ol in the chromatogram obtained with test solution (b) (300 ppm), -- disregard limit: 0.03 times the area of the peak due to 4-methylpentan-2-ol in the chromatogram obtained with test solution (b) (9ppm). | BP 2023 |
|  | Residue on evaporation | The residue weighs a maximum of 2.5 mg | BP 2023 |
|  | Ethanol content | 95.1 %V/V - 96.9 %V/V | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Store in a well - closed container and protected from light.