| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | A. Thin-layer chromatography. Test solution To 1.0 g add 25 mL of a mixture of equal volumes of hydrochloric acid R and water R and heat on a water-bath for 15 min. Cool and filter. Use the filtrate for identification test C. Wash the residue with 50 mL of a mixture of 1 volume of dilute hydrochloric acid R and 9 volumes of water R and discard the washings. Dry the residue at 100-105 °C. Dissolve 20 mg of the residue in acetone R and dilute to 10 mL with the same solvent. Reference solution Dissolve 20 mg of fumaric acid CRS in acetone R and dilute to 10 mL with the same solvent. PlateTLC silica gel F254 plate R. Mobile phaseanhydrous formic acid R, methylene chloride R, butanol R, heptane R (12:16:32:44 V/V/V/V). Application 5 µL. Development In an unsaturated tank, over a path of 10 cm. Drying At 105 °C for 15 min. Detection Examine in ultraviolet light at 254 nm. B. Mix 0.5 g with 1 g of resorcinol R. To 0.5 g of the mixture in a crucible add 0.15 mL of sulfuric acid R and heat gently. A dark red semi-solid mass is formed. Add the mass, with care, to 100 mL of water R. C. The filtrate obtained during preparation of the test solution in identification test A gives reaction (a) of iron. Heat 0.15 g with 8 mL of dilute hydrochloric acid R and 20 mL of distilled water R. Cool in iced water, filter and dilute to 30 mL with distilled water R. |
|  | SULPHATES | In a flask with a ground-glass stopper, dissolve 3.0 g in a mixture of 10 mL of hydrochloric acid R and 100 mL of water R by heating rapidly to boiling. Boil for 15 s. Cool rapidly, add 3 g of potassium iodide R, stopper the flask and allow to stand protected from light for 15 min. Add 2 mL of starch solution R as indicator. Titrate the liberated iodine with 0.1 M sodium thiosulfate. Carry out a blank test. The difference between the volumes used in the 2 titrations corresponds to the amount of iodine liberated by ferric ion. 1 mL of 0.1 M sodium thiosulfate is equivalent to 5.585 mg of ferric ion. |
|  | FERRIC IRON | Atomic absorption spectrophotometryTest solution Dissolve 0.500 g in a mixture of 2.5 mL of hydrochloric acid R and 20 mL of water R, heating slightly if necessary. Allow to cool, filter if necessary and dilute to 25.0 mL with water R. Dilute 1.0 mL of the solution to 10.0 mL with water R. |
|  | ZINC | Reference solutions Prepare the reference solutions using zinc standard solution (10 ppm Zn) R and diluting with a 1 per cent V/V solution of hydrochloric acid R. Source Zinc hollow-cathode lamp. Wavelength 213.9 nm. |
|  | LOSS ON DRYING | Determined on 1.000 g by drying in an oven at 105 °C. |
|  | ASSAY | Dissolve with slight heating 0.150 g in 7.5 mL of dilute sulfuric acid R. Cool and add 25 mL of water R. Add 0.1 mL of ferroin R. Titrate immediately with 0.1 M cerium sulfate until the colour changes from orange to light bluish-green. 1 mL of 0.1 M cerium sulfate is equivalent to 16.99 mg of C4H2FeO4. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |