| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe colour and other characteristics of the material. |
|  | SOLUBILITY | Determine solubility of the material in water, in ethanol(96%), in fatty oils and in essential oils. |
|  | IDENTIFICATION | First identification A, B. Second identification A, C. A. Determine the refractive index of the material. B. Infrared absorption spectrophotometry. Preparation To 5 ml add 1 mL of water R and mix carefully Comparison Ph. Eur. Reference spectrum of glycerol (85%) C. Determine the Relative density of the material ( 1.258 to 1.268.) |
|  | APPEARANCE OF SOULTION | SOLUTION S: Dilute 100.0 g to 200.0 mL with carbon dioxide – free water R. Observe the clarity of solution S. Dilute 10 mL of solution S to 25 mL with water R. Note its colour intensity. |
|  | ACIDITY OR ALKALINITY | To 50 mL of solution S, add 0.5 mL of phenolphthalein solution R. The solution is colourless. Titrate with 0.1 M sodium hydroxide to change the colour of the indicator to pink. |
|  | REFRACTIVE INDEX | Determine the refractive index. |
|  | ALDEHYDES | Place 7.5 mL of solution S in a ground glass stoppered flask and add 7.5 mL of water R and 1.0 mL of decolorized pararosaniline solution R. Close the flask and allow to stand for 1 hr at a temperature of 25 + 1 0C. The absorbance of the solution measured at 552 nm is not greater than that of a standard prepared at the same time and in the same manner using 7.5 mL of formaldehyde standard solution ( 5 ppm CH20) R and 7.5 mL of water R. The test is not valid unless the standard is pink. |
|  | ESTERS | Add 10.0 mL of 0.1 M sodium hydroxide to the final solution obtained in the test for acidity or alkalinity. Boil under a reflux condenser for 5 minutes. Cool. Add 0.5 mL of phenolphthalein solution R and titrate with 0.1 M hydrochloric acid. |
|  | IMPURITY A AND RELATED SUBSTANCES | Gas chromatography. Test solution Dilute 10.0 mL of solution S to 100.0 mL with water R. Reference solution (a) Dilute 10.0 g of glycerol R1 to 20.0 mL with water R. Dilute 10.0 mL of the solution to 100.0 mL with water R. Reference solution (b) Dissolve 1.000 g of diethylene glycol R in water R and dilute to 100.0 mL with the same solvent. Reference solution (c) Dilute 1.0 mL of reference solution (b) to 10.0 mL with reference solution (a). Dilute 1.0 mLof this solution to 20.0 mL with reference solution (a). Reference solution (d) Mix 1.0 mL of the test solution and 5.0 mL of reference solution (b) and dilute to 100.0 mL with water R. Dilute 1.0 mL of this solution to 10.0 mL with water R. Reference solution (e) Dilute 5.0 mL of reference solution (b) to 100.0 mL with water R. Column: -- size: l = 30 m, Ø = 0.53 mm, -- stationary phase: 6 % polycyanopropylphenylsiloxane and 94% of polydimethylsiloxane Carrier gas helium for chromatography R Split ratio 1: 10 Linear velocity 38 cm/s Temperature: Time (min); Temp.(0C). column; 0 0 -- 16 16 -- 20; 100 100 – 220 220. Injection port; 220. detector; 250. Detection Flame ionization Injection 0.5 µL Elution order Impurity A, glycerol System suitability Reference solution (d): --- resolution: minimum 7.0 between the peaks due to impurity A and glycerol. Limits : -- impurity A: not more than the area of the corresponding peak in the chromatogram obtained with reference solution (c) (0.1 %), -- any other impurity with a retention time less than the retention time of glycerol: not more than the area of the peak due to impurity A in the chromatogram obtained with reference solution (c ) (0.1 %), -- total of all impurities with retention times greater than the retention time of glycerol: not more than 5 times the area of the peak due to impurity A in the chromatogram obtained with reference solution ( c) (0.5 %), -- disregard limit: 0.05 times the area of the peak due to impurity A in the chromatogram obtained with reference solution (e) ( 0.05 %). |
|  | HALOGENATED COMPOUND | To 10 mL of solution S add 1 mL of dilute sodium hydroxide solution R, 5 mL of water R, and 50 mg of halogen-free nickel-aluminium alloy R. Heat on a water bath for 10 min, allow to cool and filter. Rinse the flask and the filter with water R until 25 mL of filtrate is obtained. To 5 mLof the filtrate add 4 mL of ethanol (96%) R, 2.5 mL of water R, 0.5 mL of nitric acid R and 0.05 mL of silver nitrate solution R2 and mix. Allow to stand for 2 min. |
|  | SUGARS | To 10 mL of solution S add 1 mLl of dilute sulphuric acid R and heat on a water bath for 5 min. Add 3 mL of carbonate – free dilute sodium hydroxide solution R (prepared by the method described for carbonate – free 1 M sodium hydroxide), mix and add dropwise 1 mL of freshly prepared copper sulphate solution R. The solution is clear and blue. Continue heating on the water bath for 5 min. The solution remains blue and no precipitate is formed. |
|  | CHLORIDES | Dilute 1 mL of solution S to 15 mL with water R and subject to the limit test for chlorides. Prepare the standard using 1 mL of chloride standard solution (5 ppm Cl) R diluted to 15 mL with water R. |
|  | WATER | Determine the water content of the material on 1.000 g. |
|  | SULPHATED ASH | Determine the sulphated ash on 5.0 g after heating to boiling and ignition. |
|  | ASSAY | Thoroughly mix 0.075 g with 45 mL of water R. Add 25.0 mL of a mixture of 1 volume of 0.1 M sulphuric acid and 20 volumes of 0.1 M sodium periodate. Allow to stand protected from light for 15 minutes. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |