| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, in acetone, in methanol, in methylene chloride and dilute solutions of alkali hydroxides and carbonates. |
|  | IDENTIFICATION | First identification: A, C. Second identification: A, B, D. A.Determine the Melting point B.Ultraviolet and visible absorption spectrophotometry Test solution Dissolve 50.0 mg in a 4 g/L solution of sodium hydroxide R and dilute to 100.0 mL with the same alkaline solution. Spectral range 240-300 nm, using a spectrophotometer with a band width of 1.0 nm and a scan speed of not more than 50 nm/min. Absorption maxima At 264 nm and 272 nm. Shoulder At 258 nm. Absorbance ratio: — A264 / A258 = 1.20 to 1.30; — A272 / A258 = 1.00 to 1.10. C. Infrared absorption spectrophotometry. Comparisonibuprofen CRS D.Thin-layer chromatography. Test solution Dissolve 50 mg of the substance to be examined in methylene chloride R and dilute to 10 mL with the same solvent. Reference solution Dissolve 50 mg of ibuprofen CRS in methylene chloride R and dilute to 10 mL with the same solvent. PlateTLC silica gel plate R. Mobile phaseanhydrous acetic acid R, ethyl acetate R, hexane R (5:24:71 V/V/V). Application 5 µL. Development Over a path of 10 cm. Drying At 120 °C for 30 min. Detection Lightly spray with a 10 g/L solution of potassium permanganate R in dilute sulfuric acid R and heat at 120 °C for 20 min; examine in ultraviolet light at 365 nm. |
|  | APPEARANCE OF SOULTION | ### Solution S Dissolve 2.0 g in methanol R and dilute to 20 mL with the same solvent. Using identical tubes of colourless, transparent, neutral glass with a flat base and an internal diameter of 15 mm to 25 mm, compare the liquid to be examined with water R or the solvent or the reference solution (see Tables of reference solutions) prescribed in the monograph, the depth of the layer being 40 mm. Compare the colours in diffused daylight, viewing vertically against a white background. |
|  | OPTICAL ROTATION | Dissolve 0.50 g in methanol R and dilute to 20.0 mL with the same solvent |
|  | RELATED SUBSTANCES | Liquid chromatography. Test solution Dissolve 20 mg of the substance to be examined in 2 mL of acetonitrile R and dilute to 10.0 mL with mobile phase A. Reference solution (a) Dilute 1.0 mL of the test solution to 100.0 mL with mobile phase A. Dilute 1.0 mL of this solution to 10.0 mL with mobile phase A. Reference solution (b) Dilute 1.0 mL of ibuprofen impurity B CRS to 10.0 mL with acetonitrile R (solution A). Dissolve 20 mg of ibuprofen CRS in 2 mL of acetonitrile R, add 1.0 mL of solution A and dilute to 10.0 mL with mobile phase A. Reference solution (c) Dissolve the contents of a vial of ibuprofen for peak identification CRS (mixture of impurities A, J and N) in 1 mL of acetonitrile R and dilute to 5 mL with mobile phase A. Column: — size: l = 0.15 m, Ø = 4.6 mm; — stationary phase: end-capped octadecylsilyl amorphous organosilica polymer for chromatography R (5 µm). Mobile phase: — mobile phase A: mix 0.5 volumes of phosphoric acid R, 340 volumes of acetonitrile R1 and 600 volumes of water for chromatography R; allow to equilibrate and dilute to 1000 volumes with water for chromatography R; — mobile phase B: acetonitrile R1; Time (min); Mobile phase A (per cent V/V); Mobile phase B (per cent V/V). 0 - 25; 100; 0. 25 - 55; 100 → 15; 0 → 85. 55 - 70; 15; 85. Flow rate 2 mL/min. Detection Spectrophotometer at 214 nm. Injection 20 µL. Identification of impurities Use the chromatogram supplied with ibuprofen for peak identification CRS and the chromatogram obtained with reference solution (c) to identify the peaks due to impurities A, J and N. Relative retention With reference to ibuprofen (retention time = about 21 min): impurity J = about 0.2; impurity N = about 0.3; impurity A = about 0.9; impurity B = about 1.1. System suitability Reference solution (b): — peak-to-valley ratio: minimum 1.5, where Hp = height above the baseline of the peak due to impurity B, and Hv = height above the baseline of the lowest point of the curve separating this peak from the peak due to ibuprofen. If necessary, adjust the concentration of acetonitrile in mobile phase A. Gas chromatography: use the normalisation procedure. |
|  | IMPURITY F | Methylating solution Dilute 1 mL of N,N-dimethylformamidedimethylacetal R and 1 mL of pyridine R to 10 mL with ethyl acetate R. Test solution Weigh about 50.0 mg of the substance to be examined into a sealable vial, dissolve in 1.0 mL of ethyl acetate R, add 1 mL of the methylating solution, seal and heat at 100 °C in a block heater for 20 min. Allow to cool. Remove the reagents under a stream of nitrogen at room temperature. Dissolve the residue in 5 mL of ethyl acetate R. Reference solution (a) Dissolve 0.5 mg of ibuprofen impurity F CRS in ethyl acetate R and dilute to 10.0 mL with the same solvent. Reference solution (b) Weigh about 50.0 mg of ibuprofen CRS into a sealable vial, dissolve in 1.0 mL of reference solution (a), add 1 mL of the methylating solution, seal and heat at 100 °C in a block heater for 20 min. Allow to cool. Remove the reagents under a stream of nitrogen at room temperature. Dissolve the residue in 5 mL of ethyl acetate R. Column: — material: fused silica; — size: l = 25 m, Ø = 0.53 mm; — stationary phase: macrogol 20 000 R (film thickness 2 µm). Carrier gashelium for chromatography R. Flow rate 5.0 mL/min. Temperature: — column: 150 °C; — injection port: 200 °C; — detector: 250 °C. Detection Flame ionisation. Injection 1 µL of the test solution and reference solution (b). Run time Twice the retention time of ibuprofen. System suitability: — relative retention with reference to ibuprofen (retention time = about 17 min): impurity F = about 1.5. |
|  | LOSS ON DRYING | Determine the Loss on drying on 1.000 g by drying in vacuum |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample. |
|  | ASSAY | Dissolve 0.450 g in 50 mL of methanol R. Add 0.4 mL of phenolphthalein solution R1. Titrate with 0.1 M sodium hydroxide until a red colour is obtained. Carry out a blank titration. 1 mL of 0.1 M sodium hydroxide is equivalent to 20.63 mg of C13H18O2. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |