| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe colour and other characteristics of the material. |
|  | IDENTIFICATION | A. Triturate the material with about an equal weight of camphor, of chloral hydrate, or of phenol and note your observation. B. It meets the requirements in Specific Tests for Optical Rotation. |
|  | MELTING RANGE OF I-MENTHOL | Determine the melting point of the material. |
|  | CONGEALING RANGE OF dl-MENTHOL | Perform this test preferably in a room having a temperature below 300C and a relative humidity below 50%. Place about 10 g of racemic menthol, previously dried in a desiccator over silica gel for 24 hours, in a dry test tube of from 18 to 20 mm internal diameter, and melt the contents at a temperature of about 400C. Suspend the test tube in water having a temperature of 230 to 250C and stir the contents of the tube continually with a thermometer, keeping the bulb of the thermometer immersed in the liquid. Racemic menthol congeals at a temperature between 27 0C and 28 0C. Shortly after the temperature has stabilized at the congealing point, add a few mg of dried racemic menthol to the congealed mass, and continue the stirring: after a few minutes the temperature of the mass quickly rises to 30.5 0C to 32.0 0C. |
|  | SPECIFIC ROTATION | Dissolve 100 mg per mL, in alcohol and determine the specific rotation. |
|  | LIMIT OF NONVOLATILE RESIDUE | Volatilize 2 g, accurately weighed, in a tared open porcelain dish on a steam bath, and dry the residue at 105 0C for 1 hour: the residue weighs not more than 1 mg (0.05 %). |
|  | CHROMATOGRAPHIC PURITY | System suitability preparation -- Dissolve decanol and menthol in ether to obtain a solution having concentrations of about 0.05 mg of each per mL. Test preparation -- Dissolve 10 mg of menthol in 50 mL of ether, and mix. Dilute 25 mL of this solution with ether to 100 mL and mix. Chromatographic system --- The gas chromatograph is equipped with a flame-ionization detector and contains a 1.8-m x 2-mm column packed with 10% phase G16 on support S1AB. The column is maintained at about 170 0C, the injection port is maintained at about 260 0C, and the detector block is maintained at about 240 0C. Dry helium is used as the carrier gas at a flow rate of about 50 mL, per minute. Chromatograph the system suitability preparation and record the peak responses as directed under Procedure: the retention time of menthol is about 0.7 relative to decanol. In a suitable chromatogram, the resolution , R of the 2 peaks is not less than 2.5, and the relative standard deviation of the ratio of the peak response obtained with menthol to that obtained with decanol is not more than 2 %. Procedure --- Inject about 2 µL of the test preparation into the gas chromatograph, and measure the peak responses. The peak response due to menthol is not less than 97 % of the sum of all the peak responses, excluding any due to the other. |
|  | READILY OXIDIZABLE SUBSTANCES IN dl-MENTHOL | Place 500 mg of dl – menthol in a clean, dry test tube, add 10 mL of a solution of potassium permanganate, prepared by diluting 3 mL of 0.1 N potassium permanganate with water to 100 mL and place the test tube in a beaker of water at a temperature between 45 0C and 50 0C. Remove the tube from the bath at intervals of 30 seconds, and mix quickly by shaking: the purple colour of potassium permanganate is still apparent after 5 minutes. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |