| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, in ethanol (96 per cent) and in methanol. |
|  | IDENTIFICATION | ### Solution S Dissolve 1.0 g in ethanol (96 per cent) R and dilute to 10 mL with the same solvent. Observe the clarity of Solution S and a five fold dilution of solution S. Compare it colour intensity with reference solution BY6 |
|  | APPEARANCE OF SOLUTION | ### Solution S Dissolve 1.0 g in ethanol (96 per cent) R and dilute to 10 mL with the same solvent. Observe the clarity of Solution S and a five fold dilution of solution S. Compare it colour intensity with reference solution BY6 |
|  | ACIDITY/ALKALINITY | To 2 mL of solution S add 3 mL of ethanol (96 per cent) R, 5 mL of carbon dioxide-free water R and 0.1 mL of bromocresol green solution R. |
|  | RELATED SUBSTANCES | Liquid chromatography (2.2.29). Test solution Dissolve 50.0 mg of the substance to be examined in 2.5 mL of methanol R and dilute to 50.0 mL with the mobile phase. Dilute 10.0 mL of this solution to 100.0 mL with the mobile phase. Reference solution (a) Dissolve 5 mg of 4-hydroxybenzoic acid R (impurity A) and 5 mg of the substance to be examined in the mobile phase and dilute to 100.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 10.0 mL with the mobile phase. Reference solution (b) Dissolve 50.0 mg of methyl parahydroxybenzoate CRS in 2.5 mL of methanol R and dilute to 50.0 mL with the mobile phase. Dilute 10.0 mL of this solution to 100.0 mL with the mobile phase. Reference solution (c) Dilute 1.0 mL of the test solution to 20.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 10.0 mL with the mobile phase. Column: — size: l = 0.15 m, Ø = 4.6 mm; — stationary phase: end-capped octadecylsilyl silica gel for chromatography R (5 µm). Mobile phase 6.8 g/L solution of potassium dihydrogen phosphate R, methanol R (35:65 V/V). Flow rate 1.3 mL/min. Detection Spectrophotometer at 272 nm. Injection 10 µL of the test solution and reference solutions (a) and (c). Run time 5 times the retention time of methyl parahydroxybenzoate. Relative retention With reference to methyl parahydroxybenzoate (retention time = about 2.3 min): impurity A = about 0.6. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample. |
|  | ASSAY | To 1.000g add 20.0ml of 1M sodium hydroxide. Heat at about 700C for 1 hour. Cool rapidly in an ice bath. Prepare a blank in the same manner. Carry out the titration on the solutions at room temperature. Titrate the excess sodium hydroxide with 0.5M sulphuric acid, continuing the titration until the second point of inflexion.. 1ml of 1M sodium hydroxide is equivalent to 152.1mg of C8H8O3 |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |