MATERIAL NAME: METRONIDAZOLE BENZOATE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or slightly yellowish, crystalline powder or flakes. | BP 2023 |
|  | Solubility | Practically insoluble in water, freely soluble in methylene chloride, soluble in acetone, slightly soluble in alcohol. | BP 2023 |
|  | Identification | A. 99 °C to 102 °C B. The solution shows two absorption maxima, at 232 nm and 275 nm. The specific absorbance at the absorption maximum at 232 nm is 525 to 575. C. IR spectrophotometry, comparing the spectrum obtained with that of Ph.Eur. reference spectrum of metronidazole benzoate. D. The solution gives the reaction of primary aromatic amines. | BP 2023 |
|  | Appearance of solution. | The solution is not more opalescent than reference suspension II and not more intensely coloured than reference solution GY3. | BP 2023 |
|  | Acidity | Not more than 0.25 mL of 0.02 M sodium hydroxide is required to change the colour of the indicator. | BP 2023 |
|  | Related substances | Limits: --impurities A, B, C: for each impurity, not more than the area of the corresponding peak in the chromatogram obtained with reference solution (b) (0.1 %). -- any other impurity: for each impurity, not more than the area of the principal peak in the chromatogram obtained with reference solution (a) ( 0.1 % ). -- total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution (a) (0.2 %). -- disregard limit: 0.1 times the area of the principal peak in the chromatogram obtained with reference solution (a) ( 0.01 % ). | BP 2023 |
|  | 7. Loss on drying | Maximum 0.5 %. | BP 2023 |
|  | 8. Sulphated ash | Maximum 0.1 % | BP 2023 |
|  | Assay | 98.5% to 101.0% (dried substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be properly stored in an airtight container, preferably at controlled room temperature.

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram