MATERIAL NAME: PARACETAMOL

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder. | BP 2023 |
|  | Solubility | Sparingly soluble in water, freely soluble in alcohol, very slightly soluble in methylene chloride. | BP 2023 |
|  | Identification | A. Melting point: 1680C – 1720C B. The specific absorbance at the maximum is 860 – 980. C. Infrared absorption spec, comparing with the spectrum obtained with paracetamol CRS. D. No precipitate is formed. A violet colour develops which does not change to red. E. It gives the reaction of acetyl. | BP 2023 |
|  | Related substances | Limits: -- impurity J: not more than 0.2 times the area of the corresponding peak in the chromatogram obtained with reference solution (c) ( 10 ppm). -- impurity K: not more than the area of the corresponding peak in the chromatogram obtained with reference solution ( c) (50 ppm); -- impurity F: not more than half the area of the corresponding peak in the chromatogram obtained with reference solution (d) ( 0.05 %). -- any other impurity: not more than half the area of the principal peak in the chromatogram obtained with reference solution (a) (0.05 %); -- total of other impurities: not more than the area of the principal peak in the chromatogram obtained with reference solution (a) (0.1 %); | BP 2023 |
|  | Loss on drying | Maximum 0.5 % | BP 2023 |
|  | Sulphated ash | Maximum 0.1 % | BP 2023 |
|  | Assay | 99.0 % -- 101.0 % (dried substance) | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be properly stored protected from light.

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram