| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water, in ethanol (96 per cent) and in methylene chloride. |
|  | MELTING POINT | Determine the melting point |
|  | IDENTIFICATION | First identification: A, D. Second identification: B, C, D. - Infrared absorption spectrophotometry Comparisonpromethazine hydrochloride CRS. - Identification test for phenothiazines by thin-layer chromatography use promethazine hydrochloride CRS to prepare the reference solution. - Dissolve 0.1 g in 3 mL of water R. Add dropwise 1 mL of nitric acid R. A precipitate is formed which rapidly dissolves to give a red solution, becoming orange and then yellow. Heat to boiling. - Dissolve 18 mg in 2 mL of methanol R. The solution gives reaction (a) of chlorides. Dissolve in 2 mL of water R a quantity of the substance to be examined equivalent to about 2 mg of chloride (Cl–) or use 2 mL of the prescribed solution. Acidify with dilute nitric acid R and add 0.4 mL of silver nitrate solution R1. Shake and allow to stand. A curdled, white precipitate is formed. Centrifuge and wash the precipitate with three quantities, each of 1 mL, of water R. Carry out this operation rapidly in subdued light, disregarding the fact that the supernatant solution may not become perfectly clear. Suspend the precipitate in 2 mL of water R and add 1.5 mL of ammonia R. The precipitate dissolves easily with the possible exception of a few large particles which dissolve slowly. |
|  | pH | Dissolve 1.0 g in carbon dioxide-free water R and dilute to 10 mL with the same solvent. Measure immediately after preparation. |
|  | RELATED SUBSTANCES | Liquid chromatography Prepare the solutions immediately before use and carry out the test protected from light. Solvent mixture triethylamine R, methanol R (1:1000 V/V). Test solution Dissolve 25.0 mg of the substance to be examined in the solvent mixture and dilute to 50.0 mL with the solvent mixture. Reference solution (a) Dissolve 2.5 mg of promethazine for peak identification CRS (containing impurities A, B and C) in the solvent mixture and dilute to 5 mL with the solvent mixture. Reference solution (b) Dilute 1.0 mL of the test solution to 100.0 mL with the solvent mixture. Dilute 1.0 mL of this solution to 10.0 mL with the solvent mixture. Reference solution (c) Dissolve 5.0 mg of promethazine impurity D CRS in the solvent mixture and dilute to 100.0 mL with the solvent mixture. Dilute 1.0 mL of the solution to 100.0 mL with the solvent mixture. Column: — size: l = 0.15 m, Ø = 4.6 mm; — stationary phase: end-capped octylsilyl silica gel for chromatography with embedded polar groups R (5 µm). Mobile phase Mix 20 volumes of methanol R, 30 volumes of acetonitrile R and 50 volumes of a 3.4 g/L solution of potassium dihydrogen phosphate R previously adjusted to pH 7.0 with potassium hydroxide R. Flow rate 1.0 mL/min. Detection Spectrophotometer at 254 nm. Injection 10 µL. Run time 2.5 times the retention time of promethazine. Identification of impurities Use the chromatogram supplied with promethazine for peak identification CRS and the chromatogram obtained with reference solution (a) to identify the peaks due to impurities A, B and C; use the chromatogram obtained with reference solution (c) to identify the peak due to impurity D. Relative retention With reference to promethazine (retention time = about 18 min): impurity D = about 0.2; impurity C = about 0.5; impurity B = about 1.4; impurity A = about 1.8. System suitability Reference solution (a): — resolution: minimum 2.0 between the peaks due to impurities B and A. |
|  | LOSS ON DRYING | Determined on 1.000 g by drying in an oven at 105 °C. |
|  | SULPHATED ASH | Determined on 1.0 g @ 8000C |
|  | ASSAY | Dissolve 0.250 g in a mixture of 5.0 mL of 0.01 M hydrochloric acid and 50 mL of ethanol (96 per cent) R. Carry out a potentiometric titration, using 0.1 M sodium hydroxide. Read the volume added between the 2 points of inflexion. 1 mL of 0.1 M sodium hydroxide is equivalent to 32.09 mg of C17H21ClN2S. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |