MATERIAL NAME: PROMETHAZINE HYDROCHLORIDE

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or faintly yellowish, crystalline powder. | BP 2023 |
|  | Solubility | Very soluble in water, freely soluble in ethanol (96 per cent) and in methylene chloride. | BP 2023 |
|  | Melting Point | About 222 °C, with decomposition. | BP 2023 |
|  | Identification | A.Infrared absorption spectrophotometry Comparisonpromethazine hydrochloride CRS. B. Thin Layer Chromatography It complies with the identification test for phenothiazines C. The solution becomes orange and an orange-red precipitate is formed. D. Filter-paper strip impregnated with 0.1 mL of diphenylcarbazide solution R over the opening of the test-tube turns violet-red. | BP 2023 |
|  | pH | 4.0 to 5.0 | BP 2023 |
|  | Related Substances | Limits: — correction factor: for the calculation of content, multiply the peak area of impurity A by 0.5; — impurity B: not more than 8 times the area of the principal peak in the chromatogram obtained with reference solution (b) (0.8 per cent); — impurity C: not more than twice the area of the principal peak in the chromatogram obtained with reference solution (b) (0.2 per cent); — impurity A: not more than the area of the principal peak in the chromatogram obtained with reference solution (b) (0.1 per cent); — impurity D: not more than the area of the corresponding peak in the chromatogram obtained with reference solution (c) (0.1 per cent); — unspecified impurities: for each impurity, not more than the area of the principal peak in the chromatogram obtained with reference solution (b) (0.10 per cent); — total: not more than 12 times the area of the principal peak in the chromatogram obtained with reference solution (b) (1.2 per cent); — disregard limit: 0.5 times the area of the principal peak in the chromatogram obtained with reference solution (b) (0.05 per cent). | BP 2023 |
|  | Loss on Drying | Maximum 0.5 per cent | BP 2023 |
|  | Sulphated Ash | Maximum 0.1 per cent | BP 2023 |
|  | Assay | 99.0 % to 101.0 % (dried substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram