| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and texture of material. |
|  | SOLUBILITY | Determine solubility of material in water, ethanol (96 per cent) and methanol. |
|  | IDENTIFICATION | A. Determine the melting point of the material. B. Infrared absorption spectrophotometry. Comparison propyl parahydroxybenzoate CRS C. Thin layer chromatography. Test solution (a): Dissolve 0.10 g of the substance to be examined in acetone R and dilute to 10 mL with the same solvent. Test solution (b): Dilute 1 mL of test solution (a) to 10 mL with acetone R. Reference solution (a): Dissolve 10 mg of propyl parahydroxybenzoate CRS in acetone R and dilute to 10 mL with the same solvent. Reference solution (b): Dissolve 10 mg of ethyl parahydroxybenzoate CRS in 1 mL of test solution (a) and dilute to 10 mL with acetone R. Plate TLC octadecylsilyl silica gel F254 plate R. Mobile phase: glacial acetic acid R, water R, methanol R (1:30:70 V/V/V) Application 2 µL of test solution (b) and reference solutions (a) and (b). Development Over 2/3 of the plate. Drying In air Detection Examine in ultraviolet light at 254 nm System suitability: reference solution (b): ---the chromatogram shows 2 clearly separated principal spots Results The principal spot in the chromatogram obtained with test solution (b) is similar in position and size to the principal spot in the chromatogram obtained with reference solution (a). |
|  | APPEARNACE OF SOLUTION | Solution S: Dissolve 1.0 g in ethanol (96 per cent) R and dilute to 10 mL with the same solvent. |
|  | ACIDITY | To 2 mL of solution S add 3 mL of ethanol (96 per cent) R, 5 mL of carbon dioxide – free water R and 0.1 mL of bromocresol green solution R. Titrate with 0.1 M sodium hydroxide. |
|  | RELATED SUBSTANCES | Liquid chromatography Test solution Dissolve 50.0 mg of the substance to be examined in 2.5 mL of methanol R and dilute to 50.0 mL with the mobile phase. Dilute 10.0 mL of this solution to 100.0 mL with the mobile phase Reference solution (a) Dissolve 5 mg of 4-hydroxybenzoic acid R (impurity A), 5 mg of ethyl parahydroxybenzoate R (impurity C) and 5 mg of the substance to be examined in the mobile phase and dilute to 100.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 10.0 mL with the mobile phase. Reference solution (b) Dissolve 50.0 mg of propyl parahydroxybenzoate CRS in 2.5 mL of methanol R and dilute to 50.0 mL with the mobile phase. Dilute 10 mL of this solution to 100.0 mL with the mobile phase. Reference solution (c ) Dilute 1.0 mL of the test solution to 20.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 10.0 mL with the mobile phase. Column: ---size: l = 0.15 m, Ø = 4.6 mm; --- stationary phase: end – capped octadecylsilyl silica gel for Chromatography R (5 µm) Mobile phase 6.8 g/L solution of potassium dihydrogen phosphate R, methanol R (35:65 V/V). Flow rate 1.3 mL/min Detection Spectrophotometer at 272 nm Injection 10 µL of the test solution and reference solution (a) and (c) Run time 2.5 times the retention time of propyl parahydroxybenzoate Relative retention With reference to propyl parahydroxybenzoate (retention time = about 4.5 min): impurity A = about 0.3; impurity C = about 0.7 System suitability; reference solution (a): ----resolution: minimum 3.0 between the peaks due to impurity C and propyl parahydroxybenzoate. Limits: -- correction factor: for the calculation of content, multiply the peak area of impurity A by 1.4; -- impurity A: not more than the area of the principal peak in the chromatogram obtained with reference solution (c) (0.5 %), -- unspecified impurities: for each impurity; not more than the area of the principal peak in the chromatogram obtained with reference solution (c ) (0.5 %), -- total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution ( c) (1.0 %), -- disregard limit: 0.2 times the area of the principal peak in the chromatogram obtained with reference solution (c) ( 0.1%). |
|  | SULPHATED ASH | Determine sulphated ash on 1g @ 8000C. |
|  | ASSAY | To 1.000 g add 20.0 mL of 1 M sodium hydroxide. Heat at about 70 0C for 1 hour. Cool rapidly in an ice bath. Prepare a blank in the same manner. Carry out the titration on the solutions at room temperature. Titrate the excess sodium hydroxide with 0.5 M sulphuric acid, continuing the titration until the second point of inflexion and determining the end - point potentiometrically. 1 mL of 1 M sodium hydroxide is equivalent to 180.2 mg of C10H12O3 |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |