MATERIAL NAME: PROPYL PARABEN

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or almost white, crystalline powder. | BP 2023 |
|  | Solubility | Very slightly soluble in water, freely soluble in ethanol (96 per cent) and in methanol. | BP 2023 |
|  | Identification | A. Melting Point: 96 0C – 99 0C B. IR spectrophotometry. Comparison propyl parahydroxybenzoate CRS C. The principal spot in the chromatogram obtained with test solution (b) is similar in position and size to the principal spot in the chromatogram obtained with reference solution (a). | BP 2023 |
|  | Appearance of solution | Solution S is clear and not more intensely coloured than reference solution BY6. | BP 2023 |
|  | Acidity | Not more than 0.1mL of 0.1 M sodium hydroxide is required to change the colour of the indicator to blue. | BP 2023 |
|  | Related substances | Limits: -- correction factor: for the calculation of content, multiply the peak area of impurity A by 1.4; -- impurity A: not more than the area of the principal peak in the chromatogram obtained with reference solution (c) (0.5 %), -- unspecified impurities: for each impurity; not more than the area of the principal peak in the chromatogram obtained with reference solution (c ) (0.5 %), -- total: not more than twice the area of the principal peak in the chromatogram obtained with reference solution ( c) (1.0 %), -- disregard limit: 0.2 times the area of the principal peak in the chromatogram obtained with reference solution (c) ( 0.1%). | BP 2023 |
|  | Sulphated Ash | Maximum 0.1 % | BP 2023 |
|  | Assay | 98.0 % - 102.0 % | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Should be stored in a cool, dry place away from direct source of heat and light.

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram