| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | A. Determine the Relative density of the material B. Determine the Refractive index of the material C. Determine the Boiling point of the material D. To 0.5 mL add 5 mL of pyridine R and 2 g of finely ground nitrobenzoyl chloride R. Boil for 1 min and pour into 15 mL of cold water R with shaking. Filter, wash the precipitate with 20 mL of a saturated solution of sodium hydrogen carbonate R and then with water R and dry. Dissolve in boiling ethanol (80 per cent V/V) R and filter the hot solution. On cooling, crystals are formed which, after drying at 100-105 °C, melt at 121 °C to 128 °C. |
|  | ACIDITY | To 10 mL add 40 mL of water R and 0.1 mL of bromothymol blue solution R1. The solution is greenish-yellow. Titrate with 0.1M Sodium Hydroxide. |
|  | OXIDIZING SUBSTANCES | To 10 mL add 5 mL of water R, 2 mL of potassium iodide solution R and 2 mL of dilute sulfuric acid R and allow to stand in a ground-glass-stoppered flask protected from light for 15 min. Titrate with 0.05 M sodium thiosulfate, using 1 mL of starch solution R as indicator. |
|  | REDUCING SUBSTANCES | To 1 mL add 1 mL of dilute ammonia R1 and heat in a water-bath at 60 °C for 5 min. The solution is not yellow. Immediately add 0.15 mL of 0.1 M silver nitrate and allow to stand for 5 min. |
|  | WATER | Determined on 5.00 g by the semi-micro determination of water. |
|  | SULPHATED ASH | Heat 50 g until it burns and ignite. Allow to cool. Moisten the residue with sulfuric acid R and ignite; repeat the operations. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |