| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | MELTING POINT | Determine the melting pont of the material (About 205 °C, with decomposition) |
|  | IDENTIFICATION | First identification: B, D Second identification: A, C, D - Ultraviolet and visible absorption spectrophotometry. Solution A: Dilute 1.0 mL of solution S to 50.0 mL with 0.1 M hydrochloric acid. Solution B: Dilute 1.0 mL of solution A to 100.0 mL with 0.1 M hydrochloric acid. Solution C : Dilute 1.0 mL of solution A to 100.0 mL with the potassium dihydrogen phosphate 0.025 M disodium hydrogen phosphate 0.025 M solution. Spectral ranges 250-350 nm for solution B; 220-350 nm for solution C. Absorption maxima 288-296 nm for solution B; 248-256 nm and 320-327 nm for solution C. B.Infrared absorption spectrophotometry. Comparison pyridoxine hydrochloride CRS. C. Thin-layer chromatography. Test solution Dissolve 1.0 g of the substance to be examined in water R and dilute to 10 mL with the same solvent. Dilute 1 mL of this solution to 10 mL with water R. Reference solution Dissolve 0.10 g of pyridoxine hydrochloride CRS in water R and dilute to 10 mL with the same solvent. Plate TLC silica gel G plate R. Mobile phaseconcentrated ammonia R, methylene chloride R, tetrahydrofuran R, acetone R (9:13:13:65 V/V/V/V). Application 2 µL. Development In an unsaturated tank, over a path of 15 cm. Drying In air. Detection Spray with a 50 g/L solution of sodium carbonate R in a mixture of 30 volumes of ethanol (96 per cent) R and 70 volumes of water R; dry in a current of air, spray with a 1 g/L solution of dichloroquinonechlorimide R in ethanol (96 per cent) R and examine immediately. D. Solution S gives reaction (a) of chlorides . |
|  | APPEARANCE OF SOLUTION | Determined on solution S |
|  | pH | Determined on solution S |
|  | RELATED SUBSTANCES | Liquid chromatography . Test solution Dissolve 25 mg of the substance to be examined in water R and dilute to 10.0 mL with the same solvent. Reference solution (a) Dilute 1.0 mL of the test solution to 100.0 mL with water R. Dilute 1.0 mL of this solution to 10.0 mL with water R. Reference solution (b) Dissolve 2.5 mg of pyridoxine impurity A CRS and 2.5 mg of 4-deoxypyridoxine hydrochloride R (impurity B) in water R and dilute to 10.0 mL with the same solvent. Dilute 2.0 mL of this solution to 10.0 mL with water R. Column: — size: l = 0.25 m, Ø = 4.6 mm; — stationary phase: base-deactivated end-capped octadecylsilyl silica gel for chromatography R (5 µm). Mobile phase Dissolve 2.72 g of potassium dihydrogen phosphate R in 900 mL of water R, adjust to pH 3.0 with dilute phosphoric acid R and dilute to 1000 mL with water R. Flow rate 1.0 mL/min. Detection Spectrophotometer at 210 nm. Injection 5 µL. Run time 2.5 times the retention time of pyridoxine. Identification of impurities Use the chromatogram obtained with reference solution (b) to identify the peaks due to impurities A and B. Relative retention With reference to pyridoxine (retention time = about 12 min): impurity A = about 1.7; impurity B = about 1.9. System suitability: reference solution (b): — resolution: minimum 1.5 between the peaks due to impurities A and B. |
|  | LOSS ON DRYING | Determine the Loss on drying on 1.000 g by drying in an oven at 105 °C. |
|  | SULPHATED ASH | Determine the Sulphated Ash on 1.0g of sample @ 8000C. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |