| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | A. Infrared absorption spectrophotometry. Comparisonriboflavin sodium phosphate CRS. B To 0.5 g add 10 mL of nitric acid R and evaporate the mixture to dryness. Ignite the residue until it becomes white, dissolve the residue in 5 mL of water R and filter. C. The filtrate obtained in identification test B gives reaction (b) of phosphates . Mix 1 mL of the prescribed solution with 2 mL of molybdovanadic reagent R. A yellow colour develops. |
|  | pH | Dissolve 0.5 g in carbon dioxide-free water R and dilute to 50 mL with the same solvent. |
|  | SPECIFIC OPTICAL ROTATION | Dissolve 0.300 g in 18.2 mL of hydrochloric acid R1 and dilute to 25.0 mL with water R. |
|  | IMPURITY E | To about 35 mg add 10 mL of methylene chloride R, shake for 5 min and filter. |
|  | RELATED SUBSTANCES | Liquid chromatography: use the normalisation procedure. Carry out the test protected from light. Test solution Dissolve 0.100 g of the substance to be examined in 50 mL of water R and dilute to 100.0 mL with the mobile phase. Dilute 8.0 mL of this solution to 50.0 mL with the mobile phase. Reference solution (a) Dissolve 60 mg of riboflavin CRS (impurity D) in 1 mL of hydrochloric acid R and dilute to 250 mL with water R. Dilute 4 mL of this solution to 100 mL with the mobile phase. Reference solution (b) Dissolve 10 mg of riboflavin sodium phosphate CRS in 5 mL of water R and dilute to 10 mL with the mobile phase. Dilute 8 mL of this solution to 50 mL with the mobile phase. Reference solution (c) Dilute 1.0 mL of the test solution to 100.0 mL with the mobile phase. Dilute 1.0 mL of this solution to 20.0 mL with the mobile phase. Reference solution (d) Dissolve 5 mg of riboflavin sodium phosphate for peak identification CRS (containing impurities F, G and H) in 2.5 mL of water R and dilute to 5 mL with the mobile phase. Dilute 4 mL of the solution to 25 mL with the mobile phase. Column: — size: l = 0.25 m, Ø = 4.6 mm; — stationary phase: base-deactivated end-capped octadecylsilyl silica gel for chromatography R (5 µm). Mobile phasemethanol R, 7.35 g/L solution of potassium dihydrogen phosphate R (15:85 V/V). Flow rate 2 mL/min. Detection Spectrophotometer at 266 nm. Injection 100 µL. Run time 2.5 times the retention time of riboflavin 5′-monophosphate. Identification of impurities Use the chromatogram obtained with reference solution (a) to identify the peak due to impurity D; use the chromatogram supplied with riboflavin sodium phosphate for peak identification CRS and the chromatogram obtained with reference solution (d) to identify the peaks due to impurities F, G and H. Relative retention With reference to riboflavin 5′-monophosphate (retention time = about 17 min): riboflavin 3′, 4′-diphosphate = about 0.2; riboflavin 3′, 5′-diphosphate = about 0.3; riboflavin 4′, 5′-diphosphate = about 0.5; impurity F = about 0.6; riboflavin 3′-monophosphate = about 0.7; impurity G = about 0.8; riboflavin 4′-monophosphate = about 0.9; impurity H = about 1.3; impurity D = about 2. System suitability Reference solution (b): — resolution: minimum 1.5 between the peaks due to riboflavin 4′-monophosphate and riboflavin 5′-monophosphate. |
|  | COMPOSITION | Liquid chromatography as described in the test for related substances. Identification of peaks Use the chromatogram supplied with riboflavin sodium phosphate CRS and the chromatogram obtained with reference solution (b) to identify the peaks due to the major and minor components listed under Definition. |
|  | INORGANIC PHOSPHATE | Acid molybdate solution Dilute 25 mL of a 70 g/L solution of ammonium molybdate R to 200 mL with water R. To this solution slowly add 25 mL of a 210 mL/L solution of sulfuric acid R. Ferrous sulfate solution A 100 g/L solution of ferrous sulfate R in a 4.2 mL/L solution of sulfuric acid R. Test solution Dissolve 0.150 g in water R and dilute to 50 mL with the same solvent. To 10.0 mL of the solution add 10.0 mL of acid molybdate solution and 5.0 mL of ferrous sulfate solution and mix. Reference solution Immediately before use, dilute with water R to 10 times its volume a solution containing potassium dihydrogen phosphate R equivalent to 0.440 g of KH2PO4 in 1000.0 mL. To 10.0 mL of this solution add 10.0 mL of acid molybdate solution and 5.0 mL of ferrous sulfate solution and mix. Measure the absorbance of the test solution and the reference solution at 700 nm using water R as compensation liquid. |
|  | LOSS ON DRYING | Determined on 1.000 g by drying in vacuo at 105 °C for 5 h. |
|  | ASSAY | Carry out the assay protected from light. Dissolve 0.100 g in 150 mL of water R, add 2 mL of glacial acetic acid R and dilute to 1000.0 mL with water R. To 10.0 mL of this solution add 3.5 mL of a 14 g/L solution of sodium acetate R and dilute to 50.0 mL with water R. Measure the absorbance at the absorption maximum at 444 nm. Calculate the content of C17H20N4O6 taking the specific absorbance to be 328. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |