| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Determine the colour and appearance of material |
|  | SOLUBILITY | Determine it solubility in water, in anhydrous ethanol, in methanol, in ethyl acetate, in methylene chloride, in methyl ethyl ketone and in toluene. |
|  | IDENTIFICATION | A. Infrared absorption spectrophotometry . Preparation Thin films between plates of sodium chloride R. B. Heat 0.5 g in a test-tube over a small flame until white fumes begin to appear. Invert the tube over a 2 B.nd tube containing 1 mL of a 1 g/L solution of chromotropic acid, sodium salt R in sulfuric acid R so that the fumes reach the solution. Shake the 2nd tube for about 10 s and heat on a water-bath for 5 min. The solution is violet. C. The residue obtained in the test for silica under Assay gives the reaction of silicates. Mix the prescribed quantity of the substance to be examined in a lead or platinum crucible by means of a copper wire with about 10 mg of sodium fluoride R and a few drops of sulfuric acid R to give a thin slurry. Cover the crucible with a thin, transparent plate of plastic under which a drop of water R is suspended and warm gently. Within a short time a white ring is rapidly formed around the drop of water. |
|  | ACIDITY | To 2.0 g add 25 mL of a mixture of equal volumes of anhydrous ethanol R and ether R, previously neutralised to 0.2 mL of bromothymol blue solution R1, and shake. |
|  | DEFOAMING ACTIVITY | Foaming solution Dissolve 5.0 g of docusate sodium R in 1 L of water R, warm to 50 °C if necessary. Defoaming solution To 50 mL of methyl ethyl ketone R add 0.250 g of the substance to be examined, warm to not more than 50 °C with shaking. Into a 250 mL cylindrical tube about 5 cm in diameter introduce 100 mL of foaming solution and 1 mL of defoaming solution. Close tightly and fix the tube on a suitable oscillating shaker that complies with the following conditions: — 250-300 oscillations per minute; — angle of oscillation of about 10°; — oscillation radius of about 10 cm. Shake for 10 s and record the time between the end of the shaking and the instant the 1st portion of foam-free liquid surface appears. |
|  | MINERAL OILS | Place 2.0 g in a test-tube and examine in ultraviolet light at 365 nm. The fluorescence is not more intense than that of a solution containing 0.1 ppm of quinine sulfate R in 0.005 M sulfuric acid examined in the same conditions. |
|  | PHENYLATED COMPOUNDS | Test solution Dissolve 5.0 g with shaking in 10.0 mL of cyclohexane R. Spectral range 200-350 nm. Calculate the corrected absorbance using the following expression: B-C B; =; absorbance at the absorption maximum between 250 nm and 270 nm;. C; =; absorbance at 300 nm.. |
|  | VOLATILE MATTER | Determined on 1.00 g by heating in an oven at 150 °C for 2 h. Carry out the test using a dish 60 mm in diameter and 10 mm deep |
|  | ASSAY | ### Silica Heat not less than 20.0 mg to 800 °C increasing the temperature by 20 °C/min under a current of nitrogen R at a flow rate of 200 mL/min and weigh the residue (silica). ### Poly(dimethylsiloxane) Infrared absorption spectrophotometry. Test solution Place about 50 mg (E) in a screw-capped 125 mL cylindrical tube, add 25.0 mL of toluene R, swirl manually to disperse and add 50 mL of dilute hydrochloric acid R, close the tube and place on a vortex mixer; shake for 5 min. Transfer the contents of the tube to a separating funnel, allow to settle and transfer 5 mL of the upper layer to a screw-capped test-tube containing 0.5 g of anhydrous sodium sulfate R. Cap and shake vigorously manually. Centrifuge to obtain a clear solution. Reference solution Introduce about 0.20 g of dimeticone CRS (poly(dimethylsiloxane)) into 100.0 mL of toluene R. Prepare the reference solution in the same way as for the test solution, using 25.0 mL of the dimeticone solution obtained above. Blank solution Shake 10 mL of toluene R with 1 g of anhydrous sodium sulfate R. Centrifuge the resulting suspension. Record the infrared absorption spectra for the test solution and the reference solution in 0.5 mm cells, from 1330 cm-1 to 1180 cm-1. Determine the absorbance of the band at 1260 cm-1. Calculate the percentage content of poly(dimethylsiloxane) using the following expression: 25×C×AM×100AE×E AM; =; absorbance of the test solution;. AE; =; absorbance of the reference solution;. C; =; concentration of the reference solution, in milligrams per millilitre;. E; =; mass of the substance to be examined, in milligrams.. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |