| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and appearance of the material. |
|  | SOLUBILITY | Determine its solubility in water and in ethanol (96 per cent). |
|  | IDENTIFICATION | A. Determine the pH of solution S. B. To 0.4 mL of iodinated potassium iodide solution R add 8 mL of distilled water R and 1 mL of solution S diluted 1 to 10 in distilled water R. Observe the colour of the solution and subject the solution to the reaction (a) of sulphates. a) Dissolve about 45 mg of the substance to be examined in 5 mL of water R or use 5 mL of the prescribed solution. Add 1 mL of dilute hydrochloric acid R and 1 mL of barium chloride solution R1. A white precipitate is formed. C. Subject solution S to the reaction (a) of sodium. a) Dissolve 0.1 g of the substance to be examined in 2 mL of water R or use 2 mL of the prescribed solution. Add 2 mL of a 150 g/L solution of potassium carbonate R and heat to boiling. No precipitate is formed. Add 4 mL of potassium pyroantimonate solution R and heat to boiling. Allow to cool in iced water and if necessary rub the inside of the test-tube with a glass rod. A dense white precipitate is formed. |
|  | APPEARANCE OF SOLUTION | Observe the clarity and colour of solution S. |
|  | pH | Determine the pH of solution S |
|  | THIOSULPHATES | To 5 mL of solution S add 5 mL of dilute hydrochloric acid R. The solution remains clear for at least 15 min. |
|  | IRON | Subject 10 mL of solution S to limit test for Iron (20 ppm). Dissolve the prescribed quantity of the substance to be examined in water R and dilute to 10 mL with the same solvent or use 10 mL of the prescribed solution. Add 2 mL of a 200 g/L solution of citric acid monohydrate R and 0.1 mL of thioglycollic acid R. Mix, make alkaline with ammonia R and dilute to 20 mL with water R. Prepare a standard in the same manner, using 10 mL of iron standard solution (1 ppm Fe) R. After 5 min, any pink colour in the test solution is not more intense than that in the standard. |
|  | ARSENIC | Maximum 5 ppm. Mix 0.20 g with 2 mL of water R in a dish. Add, drop by drop, 1.5 mL of nitric acid R. Evaporate the mixture to dryness on a water-bath. Heat over a flame until no more vapour is evolved. Take up the residue in 25 mL of water R. The apparatus (see Figure 2.4.2.-1) consists of a 100 mL conical flask closed with a ground-glass stopper through which passes a glass tube about 200 mm long and of internal diameter 5 mm. The lower part of the tube is drawn to an internal diameter of 1.0 mm, and 15 mm from its tip is a lateral orifice 2 mm to 3 mm in diameter. When the tube is in position in the stopper, the lateral orifice should be at least 3 mm below the lower surface of the stopper. The upper end of the tube has a perfectly flat, ground surface at right angles to the axis of the tube. A second glass tube of the same internal diameter and 30 mm long, with a similar flat ground surface, is placed in contact with the first, and is held in position by two spiral springs. Into the lower tube insert 50 mg to 60 mg of lead acetate cotton R, loosely packed, or a small plug of cotton and a rolled piece of lead acetate paper R weighing 50 mg to 60 mg. Between the flat surfaces of the tubes place a disc or a small square of mercuric bromide paper R large enough to cover the orifice of the tube (15 mm × 15 mm). In the conical flask dissolve the prescribed quantity of the substance to be examined in 25 mL of water R, or in the case of a solution adjust the prescribed volume to 25 mL with water R. Add 15 mL of hydrochloric acid R, 0.1 mL of stannous chloride solution R and 5 mL of potassium iodide solution R, allow to stand for 15 min and introduce 5 g of activated zinc R. Assemble the two parts of the apparatus immediately and immerse the flask in a bath of water at a temperature such that a uniform evolution of gas is maintained. Prepare a standard in the same manner, using 1 mL of arsenic standard solution (1 ppm As) R, diluted to 25 mL with water R. After not less than 2 h the stain produced on the mercuric bromide paper in the test is not more intense than that in the standard. |
|  | ASSAY | Dissolve 0.200 g in 50.0 mL of 0.05 M iodine and add 5 mL of dilute hydrochloric acid R. Titrate the excess of iodine with 0.1 M sodium thiosulphate using 1 mL of starch solution R, added towards the end of the titration, as indicator. 1 mL of 0.05 M iodine is equivalent to 4.753 mg of Na2S2O5. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |