MATERIAL NAME: SORBITOL

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | Clear, colourless syrupy liquid. | BP 2023 |
|  | Solubility | Miscible with water. | BP 2023 |
|  | Identification | A. The principal peak in the chromatogram obtained with the test solution is similar in retention time to the principal peak in the chromatogram obtained with reference solution (a). B. The angle of rotation is +1.50 to +3.50. C. It is a clear, syrupy liquid at 25 0C | BP 2023 |
|  | Appearance of Solution | The solution is clear and colourless. | BP 2023 |
|  | Conductivity | Maximum 10 µS.cm -1 | BP 2023 |
|  | Related Sugars | Not less than 12.8 mL of 0.05 M sodium thiosulphate is required.( Maximum 0.2 % calculated as glucose equivalent.) | BP 2023 |
|  | Related Sugars after hydrolysis | Not less than 8.0 mL of 0.05 M sodium thiosulphate is required. Maximum 9.3 % calculated as glucose equivalent. | BP 2023 |
|  | Lead | Maximum 0.5 ppm | BP 2023 |
|  | Nickel | Maximum 1 ppm | BP 2023 |
|  | Water Content | 28.0 % to 32.0 % m/m | BP 2023 |
|  | Assay | Anhydrous substance: 68.0% m/m to 72.0 % m/m. D – glucitol (D-sorbitol, C6H1406): 72.0% to 92.0% (anhydrous substance) | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

STORAGE: Store in an airtight container.

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram