| S/N | TEST | DESCRIPTIONS OF PROCEDURE |
| --- | --- | --- |
|  | DESCRIPTION | Describe the colour and appearance of the material. |
|  | SOLUBILITY | Determine its solubility in water, in ethanol (96%) and in dilute solutions of acids and alkali hydroxides. |
|  | IDENTIFICATION | First identification A Second identification B, C. A. Examine by infrared absorption spectrophotometry. The spectrum shows absorption bands at 3677+ 2 cm-1, at 1018 + 2 cm -1 and at 669 + 2 cm -1. Examine the substance as discs prepared using potassium bromide R. B. In a platinum crucible, melt a mixture of 0.2 g of anhydrous sodium carbonate R and 2.0 g of potassium carbonate R. To the melted mass add 0.1 g of the substance to be examined and heat until the mixture is completely melted. Allow to cool and transfer the melted mass into an evaporating dish with 50 ml of hot water R. Add hydrochloric acid R until effervescence ceases. Add 10 ml of hydrochloric acid R and evaporate to dryness on a water-bath. Allow to cool. Add 20 ml of water R, heat to boiling and filter (the residue is used for identification test C). To 5 ml of the filtrate add 1 ml of ammonia R and 1 ml of ammonium chloride solution R and filter. To the filtrate add 1 ml of disodium hydrogen phosphate solution R. Note any coloured precipitate formed. C. Subject the residue obtained in identification test B to the reaction of silicates. Mix the prescribed quantity of the substance to be examined in a lead or platinum crucible by means of a copper wire with about 10 mg of sodium fluoride R and a few drops of sulfuric acid R to give a thin slurry. Cover the crucible with a thin, transparent plate of plastic under which a drop of water R is suspended and warm gently. Within a short time a white ring is rapidly formed around the drop of water. TESTS Solution S1 Weigh 10.0 g of the substance to be examined into a conical flask fitted with a reflux condenser, gradually add 50 ml of 0.5 M hydrochloric acid while stirring and heat on a water-bath for 30 mins. Allow to cool. Transfer the mixture to a beaker and allow the undissolved material to settle. Filter the supernatant through medium – speed filter paper into a 100 ml volumetric flask, retaining as much as possible of the insoluble material in the beaker. Wash the residue and the beaker with 3 quantities, each of 10 ml, of hot water R. Wash the filter with 15 ml of hot water R, allow the filtrate to cool and dilute to 100.0 ml with the same solvent. Solution S2 Perchlorates mixed with heavy metals are known to be explosive. Take proper precautions while performing this procedure. Weigh 0.5 g of the substance to be examined in a 100 ml polytetrafluoroethylene dish, add 5 ml of hydrochloric acid R, 5 ml of lead–free nitric acid R and 5 ml of perchloric acid R. Stir gently then add 35 ml of hydrofluoric acid R and evaporate slowly to dryness on a hot plate. To the residue, add 5 ml of hydrochloric acid R, cover with a watch glass, heat to boiling and allow to cool. Rinse the watch – glass and the dish with water R. Transfer into a volumetric flask, rinse the dish with water R and dilute to 50.0 ml with the same solvent. |
|  | ACIDITY OR ALKALINITY | Boil 2.5 g with 50 ml of carbon dioxide–free water R under reflux. Filter in vacuo. To 10 ml of the filtrate add 0.1 ml of bromothymol blue solution R1, not more than 0.4 ml of 0.01 M hydrochloric acid is required to change the colour of the indicator to green. To 10 ml of the filtrate add 0.1 ml of phenolphthalein solution R1 and titrate with 0.01 M sodium hydroxide to change the colour of the indicator to pink. |
|  | WATER SOLUBLE SUBSTANCES | To 10.0 g add 50 ml of carbon dioxide – free water R, heat to boiling and maintain boiling under a reflux condenser for 30 minutes. Allow to cool, filter through a medium-speed filter paper and dilute to 50.0 ml with carbon dioxide – free water R. Take 25.0 ml of the filtrate, evaporate to dryness and heat at 1050C for 1 hour. Weigh the residue obtained. |
|  | ALUMINIUM | Atomic absorption spectrometry. Test solution: To 5.0 ml of solution S2 add 10 ml of a 25.34 g/l solution of caesium chloride R, 10.0 ml of hydrochloric acid and dilute to 100.0 ml with water R. Reference solutions: Into 4 identical volumetric flasks, each containing 10.0 ml of hydrochloric acid R and 10 ml of a 25.34 g/l solution of caesium chloride R, introduce respectively 5.0 ml, 10.0 ml, 15.0 ml and 20.0 ml of aluminium standard solution (100 ppm Al) R and dilute to 100.0 ml with water R. Measure the absorbance at 309.3 nm, using an aluminium hollow-cathode lamp as the radiation source and a nitrous oxide – acetylene flame. |
|  | CALCIUM | Atomic absorption spectrometry. Test solution: To 5.0 ml of solution S2 add 10.0 ml of hydrochloric acid R, 10 ml of lanthanum chloride solution R and dilute to 100.0 ml with water R. Reference solutions Into 4 identical volumetric flasks, each containing 10.0 ml of hydrochloric acid R and 10 ml of lanthanum chloride solution R, introduce respectively 1.0 ml, 2.0 ml, 3.0 ml and 5.0 ml of calcium standard solution (100 ppm Ca) R1 and dilute to 100.0 ml with water R. Measure the absorbance at 422.7 nm using a calcium hollow-cathode lamp as the radiation source and a nitrous oxide – acetylene flame. |
|  | IRON | Atomic absorption spectrometry. Test solution To 2.5 ml of solution S1, add 50.0 ml of 0.5 M hydrochloric acid and dilute to 100.0 ml with water R. Reference solutions Into 4 identical volumetric flasks, each containing 50.0 ml of 0.5 M hydrochloric acid, introduce respectively 2.0 ml, 2.5 ml, 3.0 ml and 4.0 ml of iron standard solution (250 ppm Fe) R and dilute to 100.0 ml with water R. Measure the absorbance at 248.3 nm using an iron hollow-cathode lamp as the radiation source and an air- acetylene flame. |
|  | LEAD | Atomic absorption spectrometry. Test solution Use solution S1 Reference solutions Into 4 identical volumetric flasks, each containing 50.0 ml of 0.5 M hydrochloric acid, introduce respectively 5.0 ml, 7.5 ml, 10.0 ml and 12.5 ml of lead standard solution (10 ppm Pb) R1 and dilute to 100.0 ml with water R. Measure the absorbance at 217.0 nm using a lead hollow-cathode lamp as the radiation source and an air-acetylene flame. |
|  | MAGNESIUM | Atomic absorption spectrometry. Test solution: Dilute 0.5 ml of solution S2 to 100.0 ml with water R. To 4.0 ml of the solution, add 10.0 ml of hydrochloric acid R, 10 ml of lanthanum chloride solution R and dilute to 100.0 ml with water R. Reference solutions Into 4 identical volumetric flasks, each containing 10.0 ml of hydrochloric acid R and 10 ml of lanthanum chloride solution R, introduce respectively 2.5 ml, 3.0 ml, 4.0 ml and 5.0 ml of magnesium standard solution (10 ppm Mg) R1 and dilute to 100.0 ml with water R. Measure the absorbance at 285.2 nm using a magnesium hollow-cathode lamp as the radiation source and an air – acetylene flame. |
|  | LOSS ON IGNITION | Determine loss on ignition on 1.00 g by ignition to constant weight at 1050 – 11000C. |
|  | MICROBIAL ENUMERATION TEST | Determination of total aerobic microbial count (TAMC) and total combined yeasts and molds count (TYMC) by the pour plate method. - Slightly open the lid of the petri plate and dispense 1 ml of diluted sample in the center of the Petri plate using a sterile calibrated pipette. - Open the lid of the prepared media. (soya bean digest agar for TAMC and Sabouraud Dextrose Agar for TYMC) - Pour about 15-20ml of the sterilized molten media at the appropriate temperature above the sample. - Close the lid of the plate then mix the sample in the media properly by gently swirling the plate. The plate is generally swirled in an “S” or “8” shape. - Allow the media to completely solidify. - Simultaneously perform the negative control by plating 1ml of diluent used instead of the sample for each medium. - Incubate the plate in an inverted position under suitable incubation conditions (For TAMC Incubate the plates of Soybean Casein Digest Agar at 30°C-35°C for 48hours and For TYMC incubate the plates of Sabouraud Dextrose Agar at 20°C - 25°C for 72hours Test for Absence of Pathogen Test for E. coli: Using same dilution - Transfer 1ml of the sample into a sterile McCartney bottle containing 9ml of sterile MacConkey broth. - Incubate the MacConkey broth bottle at 300C-350C for 24 hours. - After incubation shake the tube and transfer loop full portion of MacConkey broth on the surface of the pre incubated Eosin Methylene Blue Agar petri plate. - Incubate plates at 30°C to 35°C for 48 hours. - For negative control, proceeding same as above except without sample, streak a portion of incubated sterile MacConkey broth medium on the surface of pre incubated Eosin Methylene Blue Agar. - Observe and record the presence or absence of specified microorganisms. - Include positive and negative controls in each batch of tests. - Verify the performance of media and reagents used in the analysis. Interpretation of Results - The bacterial count should not exceed 102cfu/ml while that of the fungal count should not be more than 102cfu/ml. There should be absence of fecal coliform (E coli). - If growth occurs in the negative control plate, the test is invalid, failed negative control must be investigated - The total aerobic microbial count (TAMC) is considered to be equal to the number of CFU found using casein soya bean digest agar; if colonies of fungi are detected on this medium, they are counted as part of TAMC. - The total combined yeasts/mould count (TYMC) is considered to be equal to the number of CFU found using Sabouraud dextrose agar; if colonies of bacteria are detected on this medium, they are counted as part of TYMC - When an acceptance criterion for microbiological quality is prescribed it is interpreted as follows: 101 CFU : maximum acceptable count＝20 102 CFU: maximum acceptable count＝200, 103 CFU: maximum acceptable count＝2000. |

| - CURRENT REVISION | - SUPERSEDE | - REASON FOR REVIEW |
| --- | --- | --- |
| - 00 | - NIL | - FIRST ISSUE |
| - 01 | - 00 | - Review of the procedures - Change in header and footer format |