MATERIAL NAME: TRIMETHOPRIM

SOP Ref. No.: BML-QDG-QA-105                		       		 EFFECTIVE DATE: JAN., 2024

| S/No | TESTS REQUIRED | SPECIFICATIONS | SPECIFICATION REFERENCES |
| --- | --- | --- | --- |
|  | Description | White or yellowish-white powder | BP 2023 |
|  | Solubility | Very slightly soluble in water, slightly soluble in ethanol (96 per cent). | BP 2023 |
|  | Identification | - Melting point: 199 °C to 203 °C. - The solution shows an absorption maximum at 287 nm. The specific absorbance at the absorption maximum is 240 to 250. - Infrared absorption spectrophotometry. Preparation Discs. Comparison trimethoprim CRS. - The organic layer, examined in ultraviolet light at 365 nm, shows green fluorescence. | BP 2023 |
|  | Appearance of Solution | The solution is not more intensely coloured than reference solution BY7 | BP 2023 |
|  | Related Substances | A.Liquid chromatography. — Limits: — any impurity: not more than 0.2 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.1 per cent); — total: not more than 0.4 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.2 per cent); — disregard limit: 0.04 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.02 per cent); disregard any peak corresponding to impurity H (relative retention = about 10.3). B.Liquid chromatography. — any impurity: not more than 0.2 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.1 per cent); — total: not more than 0.4 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.2 per cent); — disregard limit: 0.04 times the area of the principal peak in the chromatogram obtained with reference solution (a) (0.02 per cent); disregard any peak due to impurity B (relative retention = about 1.3). | BP 2023 |
|  | Impurity K | Gas chromatography . — impurity K: not more than the area of the corresponding peak in the chromatogram obtained with the reference solution (5 ppm). | BP 2023 |
|  | Loss on Drying | Maximum 1.0 per cent | BP 2023 |
|  | Sulphated Ash | Maximum 0.1 per cent | BP 2023 |
|  | Assay | 98.5 per cent to 101.0 per cent (dried substance). | BP 2023 |
|  | Microbial Contamination; TMC Total Yeast and Mould count Pathogens | NMT 1000cfu/g NMT 100cfu/g Must be absent | BP 2023 |

TMC= Total Microbial Count, NMT= Not More Than, cfu/g= Colony Forming Unit per Gram