- OBJECTIVE:

This SOP describes the process of validating disinfectants and cleaning solutions used in Cleaning process to assess the effectiveness of these solutions in eliminating or reducing microbial contamination.

- SCOPE:

To provide written guidelines for validating efficacy of disinfectant and cleaning solutions

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Microbiologist/Lab Assistant: Responsible for preparing solutions, performing tests, and documenting results.

- Quality Assurance (QA): Responsible for reviewing and approving the validation report.

- Quality Control Manager/Supervisor: Responsible for overall approval of the validation process.

- ABBREVIATION AND DEFINITIONS

Cfu – colony forming unit

TMC- Total Microbial Count

Conc. - Concentration

- PROCEDURES

-	Prepare the disinfectant solution according to the manufacturer's instructions and SOP guidelines.

-	Remove visible dirt’s and clean surfaces with a detergent and water.

-	Prepare a suspension of the test microorganism at a specific concentration.

-	Apply the disinfectant to the surface or material to be tested.

-	Allow the disinfectant to remain on the surface for a specified contact time (e.g., 5, 10, or 15 minutes).

-	Collect samples from the treated surface after the contact time.

-	Perform microbial analysis by plating the samples on appropriate culture media and incubating them.

-	Count the number of colonies after incubation to determine the effectiveness of the disinfectant.

-	Include control samples for comparison.

-	Record all data in a logbook or checklist, including the date, time, disinfectant, and microorganism, contact time, and results.

Evaluation and Reporting:

-	Analyze the results to determine if the disinfectant meets the predefined efficacy criteria.

-	Prepare a validation report that includes the materials, procedure, results, and conclusions.

Deviations: Identify and document any deviations from the procedure.

Frequency:

-	The frequency of validation should be determined based on the risk assessment of the area or material being disinfected.

-	Regular validation is recommended for high-risk areas, after changes to the disinfectant or cleaning procedures, or as required by regulatory bodies.

Safety Precautions:

- Always wear appropriate PPE.

- Avoid mixing disinfectants.

- Store chemicals in labeled, secure containers.

- Check for the name, active ingredient and expiry date of the disinfectant/cleaning solution.

- Note the above details in the record sheet.

Method

- Prior to cleaning, take swab of different locations in an area or section with the disinfectant/ cleaning solution under test.

- Repeat the same process above after cleaning the area or section with the disinfectant/ cleaning solution.

- Inoculate the swab taken from both sample (before and after cleaning) using appropriate media and incubate in the incubators.

- The known concentration of microorganisms is mixed with the disinfectant of respective concentration. And the recovery of the microorganisms is checked at 5 minutes, 10 minutes, and 15 minutes.

Requirements

- Disinfectant solution

- Normal saline.

- SCDA & SDA

- Sterile purified water

- Saline

- Microbial culture suspension of Environmental Isolate

- Staphylococcus aureus ATCC 6538

- Escherichia coli ATCC 8739

- Pseudomonas aeruginosa ATCC 9027

SET-1: Prepare the 100 ml of the disinfectant solution of the respective concentration.

- Distribute the diluted disinfectant solution in 20 ml quantities in 4 different sterile test tubes, one tube for each microorganism.

SET-2: Prepare another set of 4 tubes using the 20 ml of sterile purified water, one tube for each microorganism.

- To the set 2 add the 0.1 ml of the suspension of the respective microorganism to the respective tube which will give the final concentration of 1x105 cfu/ml to 1x106 cfu/ml.

- Carry out the10 fold serial dilution of each microorganism up to 10-6 using 9 ml normal saline.

- And then plate out1 ml of each dilution.

- To this add 20 ml of sterile molten cooled at 45° C Soybean casein digest agar  for bacterial cultures and 20 ml of sterile molten cooled at 45° C Sabouraud dextrose agar for fungi.

- To the set 1 add the 0.1 ml of the suspension of respective Microorganism to the respective tube which will give the final concentration of 1x105 cfu/ml to 1x106 cfu/ml same as the set2.

- Carry out the10 fold serial dilution of each microorganism from the respective tube of set 1 up to 10-6 using 9 ml normal saline after 5 minutes, 10 minutes and after 15 minutes.

- And then plate out 1 ml of each dilution.

- To this add 20 ml of sterile molten cooled at 45°C Soybean casein digest agar for bacterial cultures and 20 ml of sterile molten cooled at 45° C Sabouraud dextrose agar for fungi.

- Incubate the Soybean casein digest agar plates at 30-35° C for 5 days and Sabouraud dextrose agar plates at 20-25° C for 5 days.

- After incubation observe the plates and record the count in the annexure, calculate the % reduction for each Microorganism, record the same.

- Acceptance criteria: The respective disinfectant should show the 90 % reduction in microbial population within 15 minutes.

Materials, Equipment, Apparatus and Reagents

- Disinfectants: Include the specific disinfectants to be validated, potentially including different brands or formulations.

- Cleaning Agents: Detergents and cleaning solutions used for pre-cleaning.

- Microorganisms: Test microorganisms (bacteria, fungi, etc.) relevant to the setting.

- Culture Media: Media for growing and culturing microorganisms (e.g., Soybean Casein Digest Agar).

- Sterile Solutions: Distilled water, saline, for preparing dilutions and washing.

- Sterile Petri Dishes and Test Tubes: For culturing and dilution.

- Personal Protective Equipment (PPE): Gloves, lab coats, etc.

- Other: Cleaning cloths, mops, spray bottles, waste disposal bags.

- REFERENCE AND ENCLOSURE

ENCLOSURE 1-Disinfectant Testing Report - BML-QDG-QA-009-F01

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

- DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

- CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |