- OBJECTIVE:

To ensure that all equipment, areas, and materials are free from any remnants of the previous product before starting a new product's manufacturing or packaging operations.

- SCOPE:

The procedure is applicable to all manufacturing, processing, and packaging areas and equipment where product changeovers occur.

- RESPONSIBILITIES/ACCOUNTABILITIES:

- Production/Operations Personnel: Responsible for performing the cleaning and initial verification as per specific cleaning SOPs and filling out the line clearance checklist in the Batch Manufacturing Record (BMR).

- Quality Assurance (QA)/In-Process Quality Assurance (IPQA) Personnel: Responsible for independent verification, cross-checking the checklist, and granting the final line clearance approval.

- Department Head: Accountable for the overall compliance with the SOP.

- ABBREVIATION AND DEFINITIONS

Nil

- PROCEDURES

- Ensure all manufacturing or packaging activities for the previous product are completed.

- Remove all remaining raw materials, in-process materials, finished products, waste, and documentation related to the previous batch from the area.

- Complete and sign off all relevant sections of the previous batch's BMR/BPR and equipment logbooks.

Cleaning of Equipment and Area:

- Affix a "To Be Cleaned" status label to the equipment and area.

- Dismantle removable parts of the equipment and clean as per specific equipment cleaning SOPs

- Perform “Type B" cleaning for product changeovers which involves Major/Deep cleaning to prevent cross-contamination when changing between different products, or after a major equipment breakdown or maintenance.

Cleaning procedure: This is a wet cleaning process using a validated cleaning agent and purified water.

- Transfer dismantled parts to a designated washing area.

- Wash/dip parts in a solution of purified water with an approved cleaning agent (e.g., 0.1% w/v SLS solution) using nylon brushes or scrubbers to remove all adhering material.

- Rinse thoroughly with ample purified water to remove all traces of the cleaning agent.

- Perform a final rinse using wfi water.

- Clean the main, non-dismantled equipment parts (Clean In Place - CIP where applicable) using the same sequence of cleaning agent and purified water rinses.

- Wipe surfaces with 70% Isopropyl Alcohol (IPA) solution for sanitization if required.

- Dry all parts thoroughly with a dry lint-free cloth and hang them for complete drying.

- Visually inspect the equipment and area to ensure they are clean and dry.

Quality Control (QC) Sampling

- For product-to-product changeovers, a swab or rinse sample of the equipment's critical contact parts are required by the QC department to verify the absence of previous product residues below the specified acceptance limits. The results must meet scientifically justifiable acceptance criteria for the new product to be introduced.

- Production can only proceed after receiving a satisfactory analytical report from QC.

Verification and Documentation:

- Once cleaning is complete and approved by production personnel, intimation is sent to the QA/IPQC department for line clearance.

- QA personnel inspect the area and equipment against a detailed checklist to ensure all points are satisfactory. The checklist typically includes:

- Area and equipment are visually clean and dry.

- No materials, residues, or documentation from the previous product are present.

- Environmental conditions (temperature, humidity, differential pressure) are within specified limits.

- All status labels (e.g., "Cleaned") are correctly affixed and updated.

- Weighing balances and other instruments are clean and calibrated.

- Correct BMR/BPR and labels for the next product are available and verified.

- Personnel are wearing appropriate gowning and protective clothing.

- The equipment is labeled "Cleaned" with an expiry date for its cleanliness status (e.g., valid for 72 hours).

- Frequency: For product-to-product changeovers, after maintenance of contact parts, or after long idle periods.

Final Approval and Start of New Operation:

- If all checks are satisfactory, the QA/IPQC person signs the line clearance checklist in the BMR/BPR, officially granting clearance for the new product's activity.

- Only after this final sign-off can the new product's manufacturing or packaging process begin.

- All line clearance activities and inspection results must be thoroughly documented in the BMR/BPR for traceability and regulatory compliance.

- The documentation should include the date, time, and signatures of the personnel involved in cleaning and verification

- REFERENCE AND ENCLOSURE

In-house Method

7.0 TRAINING INVOLVEMENT

| Quality and Production Managers | Quality and Production heads |
| --- | --- |
| QA staff | Production staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | PRODUCTION |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |