- OBJECTIVE:

The purpose of this SOP is to describe the procedure for the Filling Machine to ensure filling accuracy, product quality, regulatory compliance, and minimize errors.

- SCOPE:

This SOP is applicable to cover filling machine which need to be periodically calibrated

- RESPONSIBILITIES/ACCOUNTABILITIES:

Filling Operator: run checks, follow procedures and perform calibration

Production Supervisor/ Production Pharmacist: oversee and maintain good records

IPQA officer: verify and ensure compliance

- ABBREVIATION AND DEFINITIONS

SOP – Standard Operating Procedure

IPQA – in-process Quality Assurance

- PROCEDURES

- Ensure machine is clean.

- Verify all safety guards are in place and check power supply.

- Set parameters (volume, speed) as per batch record/specs.

- Flush the solution line for 5-10min with hot WFI.

- Adjust the settings to the required volume i.e. 100ml, 500ml, 1000ml.

- Press start on the filling machine and allow solution run for 5 minutes and press stop.

- Fix the pouches on the machine and press start again.

- Fill the pouches with the solution and dispense it into the measuring cylinder to check the volume.

- If the volume is not accurate/satisfactory, readjust the settings until the required volume.

- Record all data (date, instrument details, readings, adjustments).

- QA verifies data and accuracy.

- Apply a "CALIBRATED" label.

- If calibration fails, stop using the machine and apply an "UNDER MAINTENANCE" label.

- Report critical failures to Head QA for investigation (as per incident SOP).

Frequency: To be done for every batch

IPQA: To check the filled volume of a product every 15 minutes.

- REFERENCE AND ENCLOSURE

ENCLOSURE 1- Filling Machine Calibration Log - BML-QDG-QA-035-F01

7.0 TRAINING INVOLVEMENT

| QA and Production managers | Production Pharmacist and Supervisor |
| --- | --- |
| QA Unit staff | Filling Room staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |