- OBJECTIVE:

- To ensure that incidents and deviations are reported, investigated, documented, and corrected.

- To outline the method for handling incidents and deviations from established written procedures and processes or deviations from established specifications.

- SCOPE:

- This Procedure applies to the incidents and deviations relating to manufacturing processes/ operations, facilities including equipment, utilities and computer systems, storage, distribution, purchasing, analytical and marketing processes in BIOMEDICAL Ltd., Ilorin.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Departmental Representatives

- Responsible for the implementation of this SOP.

- Ensure that all incidences and/or on conformances are reported.

Documentation:

- To Issue form and ensure proper documentation.

- Shall ensure adequate implementation of corrections and corrective actions.

- To participate in the evaluation the impact of deviation/incidence on the product.

- To ensure all incidences and deviations are investigated; correction/ corrective actions instituted and followed up for completion.

HODs:

- To ensure strict compliance of the departmental representatives on the requirement of this procedure.

- Ensure all incidences and deviations are investigated and corrected.

- To participate in the evaluation the impact of deviation/incidence on the product.

QA Manager: Shall approve all deviations/incident reports, correction/ corrective/ actions plan.

- DEFINITIONS AND ABBREVIATIONS:

- CAPA: Corrective Action and Preventive Action

- QA: Quality Assurance.

- Incident: An event that is against the cGMP but not described in an official document such as SOP, Batch Manufacturing Record, methods.

- Deviations: An event that has been performed differently and/or modified differently from approved SOPs, methods, Specifications, Protocols, Good Manufacturing Practice (GMP), Batch Records, or other official documentation.

- Planned Deviation: Are deviation which are pre-approved and different from the current documents/SOPs or systems and are to be implemented for a period or for specific number of batches.

- Un-Planned Deviation: Are deviations which are not pre- approved and usually occur unexpectedly. Also an accidental and unanticipated non-conformance or deviation observed during or after the execution of activity.

- Major Deviation: Process parameter out of specification; product quality, purity or safety in question.

- Critical Deviation: When the deviation affects a quality attribute, a critical process parameter, an equipment or instrument critical for process or control, of which the impact to patients, personnel or environment is highly probable, including life threatening situation Examples: incorrect reagent or concentration used; product mislabeled; supplier management system failure.

- Other Deviations: When the deviations do not affect any quality attribute, a critical process parameter, or an equipment or instrument critical for process or control e.g. raw material is received in damaged container, manometer readings in the sampling booth crossed the action limit.

- Correction: Action to eliminate a detected nonconformity.

- Corrective Action: Action to eliminate the cause of a detected nonconformity or other undesirable situation.

- Preventive Action: Action to eliminate the cause of a potential nonconformity or other undesirable situation.

- Nonconformity: Nonfulfillment of a requirement.

- Defects: A non-fulfillment of an intended usage requirement of reasonable expectation including one concerned with safety.

- PROCEDURE

All deviations must be documented, investigated, traced, and trended. All deviation shall be reported as when they occur.

Investigations are to be carried out for the events that arise from the following:

- Failure to follow GMP procedure, Staff Observation, housekeeping deviation, Trending Data, Process Performance Monitoring, Material complaint, Product processing (manufacturing/packaging), Bulk/end-time/spot checking of products, Batch records review. Deviations and incidences can be captured and investigated in the following steps:

Step 1: Report the Event

- Observer of the event notifies the sectional head/line leader where the event occurred.

- First Line Supervisors must ensure that any immediate action required to contain the event is implemented such as stopping operations, segregating materials or product in anticipation of quarantine by QA, etc.

- The First Line Supervisors then reports the event to the QA Documentation officer and records the event in the respective Section or Department’s Incident/Deviation Logbook.

| S/N | Date of Occurrence | Time of Occurrence | Description of Event | Immediate Action | Reported B |
| --- | --- | --- | --- | --- | --- |

- The QA Documentation officer issues a Deviation Alert Form [BML/QDG/QA/058/F01] to the Departmental supervisor of the location where the event occurred.

- The record of the event must be clear, concise, and include all details relevant to the event and available to the observer.

- The alert form is filled out immediately and returned to the QA Documentation officer.

- The event is assigned a unique reference number and entered the Deviation or Incident Logbook respectively LGB/QA/058/DO/04 and

QA Documentation Deviation Logbook LGB/QA/058/DO/04

| Date | Deviation Number | Description of Event | Immediate Action | Classification of Event | Outcome / Verification | Date closed |
| --- | --- | --- | --- | --- | --- | --- |

And	QA Documentation Incident Logbook LGB/QA/058/DO/05

| Date | Incident Number | Description of Event | Immediate Action | Classification of Event | Outcome / Verification | Date closed |
| --- | --- | --- | --- | --- | --- | --- |

Assigning Deviation Number:

- The Quality Assurance Manager or designate issues a unique identification number BML/IR or DR/PDF/YY/XXX e.g.: BML/DR/PDF/00/001 or BML/IR/PDS/00/001

Where: 
BML = BIOMEDICAL LTD. 
DR = Deviation Report

IR = Incident Report
PDF =Production infusion, PDS =Production Syrup, ENG= Engineering, QCF= Quality Control infusion, QAF= Quality Assurance infusion, WHF= Materials/Warehouse infusion, QCS= Quality Control Syrup, QAS= Quality Assurance Syrup, WHS= Materials/Warehouse Syrup, S&M= Sales& Marketing, RA= Regulatory Affairs, ADM= Human Resources/Admin, PI= Production Infusion.
00 = Year 2000
001 = the first report for the section in the year

- The QA Documentation officer then issues the Incident or Deviation Form Assessment of BML/QDG/QA/058/F04 or BML/QDG/QA/058/F02 to the responsible Departmental Manager.

Step 2: Assess the Event

- The Incident or Deviation Form Assessment of BML/QDG/QA/058/F04 or BML/QDG/QA/058/F02, issued with a unique reference number assigned to the event is filled with the details relevant to the event. It is required that the recording of the event be timely. The type of the event is indicated on the form.

- This step involves categorization of the event into Incident and Deviation, risk assessment of the impact of the event and classification of the event using the procedure stated in the form Incident or Deviation Assessment BML/QDG/QA/058/F04 or BML/QDG/QA/058/F02

Events Categorization (BML/QDG/QA/058/C01)

- Types of events: Incidents; Other Deviations; Critical Deviations; Major Deviations and Planned Deviations or unplanned deviations

- Carry out assessment to determine event type anchored on the following questions:

      - Can the event affect a product attribute, manufacturing operational parameter or the product’s quality?

      - Does the event contradict or omit a requirement or instruction contemplated in any kind of approved written procedure or specification?

      - Impact to patients (or personnel or environment) is highly probable, including life threatening situation,

      - Has event exceeded the maximum number of repeats allowed (5 per annum)?

      - Are the corrective actions to be taken described in a procedure?

Incidents: If answers to all the above questions are NO, the event may be considered an incident.

Examples of incidents include the followings:

- Temporary power failure in a warehouse where no temperature sensitive materials are stored, with no temperature excursion from the established range.

- Production process parameters or environmental monitoring data reach alert levels but are still within acceptable range.

- Someone is found wearing comfort zone shoes in controlled zone area,

- Spillage of material on the floor.

If the answer to any of the questions above is YES, it would be handled as a deviation as indicated below.

Other Deviations:

- When the deviation does not affect any quality attribute, a critical process parameter, or an equipment or instrument critical for process or control, it would be categorized as Other Deviation. Examples of Other Deviations are as below:

- Skip of FEFO principle (first expired-first out) in raw material handling.

- Balance out of tolerance used to determine gross weight of raw materials upon reception.

- Inadequately trained personnel to perform warehouse cleaning activities.

- Minor errors in batch records or document that did not affects the integrity of the data.

- Spillage of material during dispensing.

- Failure to meet environmental condition during packaging.

Major Deviations:

- When the deviation affects a quality attribute, a critical process, equipment or instrument critical for process or control, of which the impact to patients (or personnel/environment) is unlikely; the deviation is categorized as ‘Major’ requiring immediate action, investigation. The deviation data base is consulted to see how many times it has happened. If the specified maximum number of repeats allowed (5 per annum) has not been exceeded, the event stays in the Major Deviation category. The appropriate corrective measures are applied, and the deviation is closed as an isolated occurrence. Examples of Major deviations include the following:

- Use of unapproved reference standard to test an API or drug product.

- Inadequately trained personnel to perform analysis.

- Production started without line clearance.

- Gross misbehavior of staff in a critical process.

- Pressure differential, temperature, and relative humidity out of established limits.

- Operational parameter out of range for a parameter defined as non-critical.

- Untrained personnel responsible for segregating the approved and rejected raw material in the warehouse.

- Equipment breakdown during activities running.

Critical Deviations:

- When the deviation affects a quality attribute, a critical process parameter, an equipment or instrument critical for process or control, of which the impact to patients (or personnel or environment) is highly probable, including life threatening situation, the deviation is categorized as critical requiring immediate action, investigated, and documented. The event is placed in this category when the answers given in the decision tree point to a ‘Critical Deviation’.  If it was first classified as ‘Major Deviation’, but has exceeded the maximum number of repeats allowed (5 per annum).

Examples of critical deviations include:

- expired or rejected API component used

- Product mislabeled.

- supplier management failure

- improperly trained personnel handling critical process e.g. sterile preparation

- use of a cleaning process that is not validated

- Filter integrity failure.

- Cross contamination or product mix-up.

Impact Assessment

- Use of FMEA-based risk analysis tool to classify and handle deviations.

- The severity of the event’s consequences and the probability of non-detection are assessed to determine the risk level.

- Depending on the risk level, events are classified as critical deviations, major deviations or incidents, and the corresponding actions are taken. The analysis is performed by reference to the assessment scales shown below:

| Criteria For Assessing Severity And Probability Of Non-Detection Of An Event |  |  |
| --- | --- | --- |
| Risk | Score | Assessment |
| Severity | 5 | The deviation affects a critical product quality attribute or parameter |
|  | 3 | The deviation affects a process, attribute or parameter that is not critical to product quality |
|  | 1 | The deviation does not affect product quality or a documented process but must be documented |
| Probability of non-detection | 5 | No controls are carried out |
|  | 3 | Final control on completed process or on manufactured product only |
|  | 1 | In-process control or other controls in place |

Key:

- If answer to all is NO = Incident

- If there is one or more YES = Deviation                        Category: ……………………..

| Risk Matrix |  | Probability of non-detection |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5 | 3 | 1 |
| Severity | 5 | 25 (RED) | 15 (RED) | 5 (ORANGE) |
|  | 3 | 15 (RED) | 9 (ORANGE) | 3 (GREEN) |
|  | 1 | 5 (ORANGE) | 3 (GREEN) | - (GREEN) |

- Risk level > 9 (red zone): Critical deviation. Requires in-depth investigation, root cause and impact assessment;

- Risk level > 3&< 9 (orange zone): Major deviation. To be investigated in proportion to the risk;

- Risk level <3 (green zone); Other Deviation / Incident. Record, To be investigated in proportion to the risk

- If the event involves materials or products, the name of the material or product and the batch/lot no are also recorded on the form. The quantity affected is also recorded.

- The Head of the Department signs off the assessment and then the Quality Assurance Documentation Head review the assessment on behalf of Quality Department.

- If there is a potential for patient or customer safety to be at risk, product efficacy to be affected, and serious regulatory compliance issues; the deviation must be treated as a priority and must be escalated.

- This assessment should be completed within 48 hours.

Step 3 and 4: Root cause Investigation and Identifying Corrective and Preventive Actions (CAPA)

- The Root Cause Investigation shall be completed using the basic risk management tool as per Quality Risk management BML-QDG-QA-083 and Corrective and Preventive Actions (CAPA) as per BML-QDG-QA-016.

- The Manager/Process owner should immediately constitute an investigation team and appoint a team leader to review the event.

- The root cause investigation can be done using any of the several tools mentioned below;

- Cause and effect diagrams  - [BML-QDG-QA-083-G01]

- Why’s analysis – [BML-QDG-QA-083-G02]

- Fishbone/Ishikawa cause and effect diagrams – [BML-QDG-QA-083-G03]

- Strength, Weakness, Opportunity, and Threat analysis (SWOT).

- Ensure the investigation tool used is part of the formal documentation supporting the deviation and investigation handling process.

- Document all steps in the investigation plan, including probable root causes that have been eliminated. The key output from this investigation must be a statement of the root cause or most probable cause of the deviation.

- Ensure every effort is made to identify the actual or most probable root cause of a significant deviation. Where this proves not to be possible, the risk associated with this uncertainty must be addressed.

- Ensure that CAPA objectives take account of this risk and include any additional requirements to control it.

- The investigation team will identify Corrective and Preventive actions to address the root cause which will be recorded.

- The root cause investigation and CAPA identification are expected to be completed within fifteen (15) working days of the recording of the event; extension of deviation could be raise by the unit head with proper justification which will be approved by the Quality Assurance Manager.

- A summary of the root cause of the event and the CAPAs identified make up the OUTCOME section of the Deviation Logbook LGB/QA/058/DO/04.

Step 5: Incident/Deviation Closure Form

- If the deviation affects a product or material, the disposition of the batch is indicated on the form.

- The head of the originating department signs off the closure form, the Quality Assurance Documentation Head reviews the closure form, while the Quality Assurance Manager approves the closure form.

Planned Deviations:

- Decision to carry out a process in a different way from which it is established in a SOP, Method, or Manufacturing Batch Record (e.g. a reprocess) due to an unforeseen event. Planned deviation is usually for a onetime event and change control to permanent changes.

- Planned deviations are documented in the form Planned Deviation Report Form BML/QDG/QA/058/F03.

- Generally continuous trending of deviations shall be carried out monthly, as well as yearly by using the monthly trend data. A copy of the trend analysis shall be forwarded to QA Head. All deviation and incident reports shall be kept in the QA Documentation unit.

- REFERENCES AND ENCLOSURE:

  - Deviation management in the context of ICH Q9/Q10

  - Incident/Deviation Alert Form [BML/QDG/QA/058/F01]

  - Deviation Assessment Report Form [BML/QDG/QA/058/F02]

  - Incident Assessment Report Form [BML/QDG/QA/058/F04]

  - Planned Deviation Report Form [BML/QDG/QA/058/F03]

  - Cause and effect guide diagram - [BML/QDG/QA/058/G01]

  - 5 Why’s analysis – [BML/QDG/QA/058/G02]

  - Fishbone/Ishikawa cause and effect diagrams – [BML/QDG/QA/058/G03]

  - Event categorization chart– [BML/QDG/QA/058/C01]

- TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0	DISTRIBUTION:

| MASTER COPIES | QUALITY |
| --- | --- |
| CONTROLLED COPIES | QUALITY, PRODUCTION, ENGINEERING, HUMAN RESOURCES/ ADMIN, STORE, SALES, AND MARKETING. |

- CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 01 | 00 | To review |
| 02 | 01 | To review |
| 03 | 02 | Inclusion of annexure into the body of the SOP |
| 04 | 03 | Broden the annexture and review |
| 05 | 04 | To update and includes all the definitions mentioned in the body of the procedure. |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |