- OBJECTIVE:

To establish a standardized procedure for the segregation, collection, handling, storage, transportation, and final disposal of all categories of waste generated in Biomedical Limited, in compliance with environmental protection guidelines.

- SCOPE:

This procedure applies to all departments within the company, including Production, Quality Control, Microbiology, Engineering, Administration, Warehouse, and Utilities, involved in generating or handling any form of waste.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Operators: Ensure segregation and initial disposal of waste at point of generation.

Supervisors: Verify proper collection, labeling, and documentation of waste.

QA Officer: Inspect disposal activities and maintain Waste Disposal Records.

Safety Officer: Ensure compliance with EHS standards.

Factory/Plant Manager: Approve and authorize disposal through registered vendors.

- ABBREVIATION AND DEFINITIONS

QA/QC – Quality Assurance / Quality Control

PPE – Personal Protective Equipment

Hazardous Waste – Waste posing risk to human health or the environment.

Biomedical Waste – Infectious, contaminated, or microbiological waste.

5.0 PROCEDURES

Destruction of Printed and Overprinted Packaging Material Rejects

- All rejected printed and overprinted packaging materials (e.g., labels, cartons, leaflets, foils, and PVC bags) shall be collected immediately after identification into a clearly labeled polyethylene bag marked “Packaging Material for Destruction.”

- The Production or Packaging Supervisor shall verify the segregation to ensure no usable or approved material is mixed with rejects.

- Using scissors or mechanical shredders, cut or shred the rejected materials into small, non-reusable pieces to prevent misuse or counterfeiting.

- Segregate the shredded materials according to type:

  - Paper-based materials (e.g., leaflets, cartons)

  - Plastic-based materials (e.g., PVC, blister film)

  - Foil-based materials (e.g., aluminum foil)

- Place the shredded materials in their respective waste bins or bags for further disposal through authorized channels — either via incineration or handover to an approved waste management vendor.

- Ensure the destruction process is carried out in the presence of a QA Officer, who must verify and endorse the destruction activity.

- Record all relevant details — including date, description, batch number, quantity, destruction method, responsible personnel, and QA witness — in the Packaging Material Destruction Logbook.

- The QA department shall retain the records for a minimum of five (5) years or as per company retention policy.

Destruction of In-Process Samples, Finished Product Samples, and Raw Materials Used for Analysis

- The Laboratory Assistant or Analyst shall collect all used or unused in-process samples, finished product samples, and raw material samples that are no longer required for testing or retention into a labeled polyethylene bag marked “Samples for Destruction.”

- Prior to destruction, verify and document the sample identity, batch number, and quantity in the Sample Disposal Register.

- For liquid samples or products in pouches:

  - Transfer the samples to the designated destruction sink or disposal area.

  - Cut open pouches or containers using a sterile blade or scissors to release the contents.

  - Allow the liquid to flow into the drain connected to the effluent treatment system (ETP).

  - Flush the drain thoroughly with copious amounts of clean water to prevent residue buildup.

- For solid or powder samples:

  - Mix the samples with sufficient water to form a slurry, ensuring dissolution before disposal.

  - Discard into the designated waste sink and flush with clean water.

- The QA Officer shall confirm completion and endorse the Destruction Approval form (BML-QDG-QA-078-F01).

- Maintain all destruction records for traceability and audit verification.

Destruction of Non-Conforming or Rejected Materials

- Materials that fail to meet specification limits during inspection or testing shall be clearly labeled as “REJECTED” and quarantined in a segregated area.

- QA shall authorize destruction only after completion of the Investigation and Material Rejection Report.

- Maintain a record of destruction containing:

  - Name of material

  - Batch/Lot number

  - Quantity destroyed

  - Date of destruction

  - Method used

  - Names and signatures of personnel involved and QA witness

Destruction Methods:

  - Liquids: Dilute the rejected liquids with an adequate quantity of water (at least 10x volume) before discharge into the effluent drainage system connected to the ETP.

  - Solids (glass, plastic, metal):

    - Break glass containers into small pieces and place them in designated sharps or scrap bins.

    - Tear or cut illegible or overprinted PVC bags and other solid waste before disposal.

    - For powdered products, make a thick slurry with water and dispose of it into the drain, followed by flushing.

- QA and Production Supervisors must jointly confirm and document that no traceable or reusable product remains after destruction.

Disposal of Contaminated Cleaning Materials (Mops, Gloves, and Tissue Papers)

- All contaminated cleaning materials such as mops, wipes, and latex gloves used during cleaning of spills, disinfection, or general sanitation shall be immediately placed in red biohazard polythene bags.

- Label each bag with “Biohazardous Waste – Contaminated Cleaning Materials.”

- Seal the bags and transfer them to the biohazard discard bins located in the controlled waste collection area.

- QA or Safety Officer shall ensure these materials are transported to the temporary waste holding area pending incineration or autoclaving.

- Maintain a Biohazard Waste Logbook indicating date, type, and source of waste, and personnel responsible for disposal.

Disposal of Biological Indicators and Sterility Test Waste

- Biological Indicators (BIs) used during sterilization validation or positive controls in microbiological studies shall be autoclaved at 121°C for 20 minutes before disposal.

- After autoclaving, transfer the inactivated BIs to the discard container for incineration or disposal through authorized waste vendor.

- Sterility Testing Media and Bottles:

  - For sterility bottles or media showing no microbial growth, add 5–10 mL of 2.5% sodium hypochlorite solution per 100 mL of media, mix gently, and pour down the drain.

  - For positive growth bottles or canisters, place them in a leak-proof container, autoclave at 121°C for 20 minutes, then disinfect as above prior to disposal.

- QA shall ensure that the autoclave cycle parameters (time, temperature, pressure) are validated and recorded in the Sterilization Logbook.

- Maintain all sterilization and disposal records for not less than five years.

Safety, Health, and Environmental Precautions

- All destruction activities must be carried out in designated, ventilated areas equipped with proper drainage and safety signage.

- Personnel must wear appropriate PPE (gloves, face mask, goggles, lab coat, safety shoes).

- Chemical residues should never be mixed with biological or general waste.

- The Safety and QA departments shall conduct periodic audits to ensure adherence to waste segregation and destruction procedures.

- Emergency eyewash stations and spill kits must be readily available near all destruction points.

- All effluents from destruction operations must pass through the Effluent Treatment Plant (ETP) before discharge to ensure compliance with NESREA and WHO standards.

- Proper safety measures and equipment must be followed during breakage of glass materials, cleaning of dust generating area

Materials, Equipment, Apparatus and Reagents

- Autoclave

- Hypochlorite

- Gloves

- Face mask

- Goggles

- Safety shoes

- Lab Coat

- REFERENCE AND ENCLOSURE

ENCLOSURE 1- Destruction Approval form -BML-QDG-QA-078-F01.

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |