| Demineralized Water | Reference: BP 2025 |
| --- | --- |
| Analysis Date: /2026 | Analysis complete Date: /2026 |
| Sampling point: DM Storage Tank | Volume sampled: 2000ml |
| Sampled by: | Analysed by: |
| Q.C Ref. No: QC/DM/26/SYP/ |  |

| S/NO | TEST | SPECIFICATION | RESULT |
| --- | --- | --- | --- |
| 1 | Description | Clear, colourless, odourless, tasteless liquid |  |
| 2 | pH | 5.5 – 7.5 |  |
| 3 | Conductivity | ≤5.1µS/cm at 25°C |  |
| 4 | Acidity/Alkalinity | ≤0.5mL of 0.01M NaOH/HCl is required to change the colour of the indicator to pink |  |
| 5 | Total Hardness | ≤5ppm |  |
| 6 | Limit test for Chlorides | ≤0.2ppm |  |
| 7 | Limit test for Sulphates | The turbidity produced in test solution for at least 1 hour should not be greater than standard solution |  |
| 8 | Limit test for Nitrates | ≤0.2ppm |  |
| 9 | Limit test for Ammonium | ≤0.2ppm |  |
| 10 | Limit test for Heavy Metals | ≤0.1ppm |  |
| 11 | Total Dissolved Solid | ≤50ppm |  |
| 12 | Residue on evaporation | ≤0.003%% |  |
| 13 | Oxidisable substances | The colour remains faintly pink |  |
| 14 | Total Bacteria count Total Yeast/ Mould count Pathogens | 102 cfu/mL 102 cfu/mL Must be absent |  |

≤ = Less Than or Equal to, ml = millilitre, CFU/ml = Colony Forming Unit per millilitre, ppm= part per million, µs/cm= microsiemens per centimetre

CHECKER’s VERDICT (box option): RELEASED / FAILED / HOLD / REPEAT Remark: