1.0 	OBJECTIVE

To lay down an approved procedure for sampling to get a representative sample of the whole lot for testing, in order to assess the critical quality parameters and release of packaging materials consignment.

2.0 	SCOPE

This SOP applies to the sampling, testing and releasing of all categories of both primary and secondary packaging materials delivered to Biomedical Limited.

3.0 	RESPONSIBILITIES/ACCOUNTABILITIES

- The quality control analyst inspects and samples the packaging material

- The assigned analyst carries out tests on the packaging material as prescribed in their individual test procedures.

- The QC analyst documents result and report any out of specification

- The Quality Control manager or designate shall review results before their approval for use.

- DEFINITION

- IPQC: In-process Quality Control

- PVC: Polyvinylchloride

- QC AR NO.: Quality Control Analysis Request number

- QA: Quality assurance

- QC: Quality control

- PROCEDURE

  - On receipt of Material intimation form BML/WHG/WH/003/F01 from Warehouse, the QC personnel acknowledge the intimation.

  - The sampler proceeds to the Packaging Material Warehouse with sampling materials such as scissors, cello tape, as well as the notification document.

  - At the point of sampling the packaging materials name, type, Lot number, quantity received, quantity sampled, supplier’s name is confirmed.

  - Ensure that the identifying labels have been affixed to all the containers/packs of the material. Check the entry on the identifying labels vis-à-vis that on the waybill for correctness. In case of any discrepancies inform Warehouse personnel in charge for corrective action.

  - The sampler checks that the materials have been delivered in good condition with no obvious damage in transit, Integrity of seal, where appropriate; and Absence of physical damage and materials correctly identified.

  - There is potential for mix up of printed packaging material during sampling and therefore only one material should be handled at the same time.

  - Sampling of PVC Pouches must be done in the clean area under laminar air flow

  - Open the package(s) carefully and observe the material for any abnormalities.

  - Primary packaging material should be protected during sampling procedures from environmental contamination.

  - Carry out random sampling as packaging materials may not necessarily be regarded as homogenous.

  - Obtain the required sample from the required number of containers, bearing in mind the sampling plan as shown in table 1

Table I

STATISTICAL SAMPLING TABLES (APPENDIX I, II, III & IV)

| BATCH SIZE (PCS) (PALLET) | APPENDIX I LEVEL I (PIECES) | APPENDIX II LEVEL II (BUNDLES) | APPENDIX III LEVEL III (REELS) | APPENDIX IV LEVEL IV (PALLET) |
| --- | --- | --- | --- | --- |
| 1-500 | 80 | 2 | 2 | 1 |
| 501-3,200 | 125 | 2 | 2 | 1 |
| 3,201-10,000 | 200 | 2 | 2 | 1 |
| 10,001-35,000 | 315 | 4 | 2 | 1 |
| 35,000,150,000 | 500 | 4 | 4 | 2 |
| 150,001-500,000 | 800 | 4 | 4 | 2 |
| 500,000-MORE | 1250 | 6 | 4 | 2 |

KEY

BOXES/PACKETS		-	LEVEL I

LEAFLETS			-	LEVEL I

LABELS				-	LEVELS II & III

BULK OUTERS			-	LEVEL II

TRAYS				-	LEVEL II

FOIL				-	LEVEL III

PVC FILM			-	LEVEL III

BOPP FILM			-	LEVEL III

ROPP CAPS			-	LEVEL I & II

BOTTLES			-	LEVELS I, II & IV

POLYTHENE BAG		-	LEVEL I

SHRINK SLEEVES		-	LEVEL I

PAPER BAGS			-	LEVEL I

- Appendix I (for cartons, leaflets, spoons, ROPP Caps, Sachet bags, Shrink sleeves and polythene bags).

- Appendix II (for outers, Self-Adhesive labels).  Sampling is done randomly from suitable number of different pallets, outers, bundles etc.

- Appendix III for Aluminum foils Self-adhesive labels, PVC film, and BOPP film.

- Appendix IV for bottles on pallets.

Note:

- Bulk outers, foils, bottles, ROPP caps, polythene bags, Shoulder trays, Self-Adhesive labels, Sachet Shrink Sleeve shall be sampled for test packing.

- Also, PVC, ROPP Caps, Measuring Cups, Bottles, Cotton Wool, Polythene Bags, and Foils are sampled for Microbiological Analysis.

  - Under Test label (Yellow) should be placed on the material and the material moved to the quarantine zone of the warehouse as the case may be.

  - Reseal opened bag of PVC/containers/bags/packs or outers and affix a completed ‘Under Test’ label to each opened container and ‘quarantine’ labels on any other container/pallet.

  - Protect the samples from external contaminants like dust, water during and after sampling.

  - Samples are taken to the laboratory and all information about the packaging material registered in the Packaging Material Sample receipt logbook (LGB/QDG/06/CLF/01) or (LGB/QDG/06/CLS/01) and issued a packaging material analytical reference number.

Packaging Material Logbook (LGB/QDG/06/CLF/01) or (LGB/QDG/06/CLS/01)

| Date | Request No. | Material name | MFD | EXD | Remark | AR. NO. | Manufacturer/ Supplier name | Analysed By |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Carry out tests on samples as prescribed in their individual standard testing procedures.

  - Results of analysis are recorded in the individual packaging material certificate of analysis Testing Record Sheet.

  - Samples of packaging material must not be returned to consignment

  - All excess packaging materials after completion of testing and reporting shall be destroyed as per the laid down procedure using destruction approval form BML/QDG/QA/078/F01

  - Keep minimum of 5 retention sample for reference of the lot supplied after approval.

- REFERENCE AND ENCLOSURE:

- Material Intimation form BML/WHG/WH/003/F01

- Destruction approval form BML/QDG/QA/078/F01

- Assurance of Pharmaceuticals. (A compendium of guidelines and related materials VOL 2).

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | Store staff |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |