- OBJECTIVE:

To provide written guidelines for Chemical Analysis of water.

- SCOPE:

This SOP covers the procedure for Chemical Analysis of water

- RESPONSIBILITY/ACCOUNTABILITY:

- QC analyst shall be responsible for testing water

- QC Heador designee approves the analytical results and releases water for production use.

4.0 ABBREVIATIONS AND DEFINITIONS

PPE– Personal Protective Equipment

- PROCEDURE

- Take a clean, dry conical flask of 1000ml in capacity, plug it with cotton wool

- Take the plugged flask to the sampling area.

- Open the valve / tap of the sampling point and let the water flow for about 30secs.

- Unplug the flask and rinse it with the running water.

- Collect about 800ml of the water sample in the flask.

- Plug the flask immediately and label it accordingly, mentioning the sampling point and date.

- Carry out chemical analysis of the water sample as per the standard operating procedures on chemical analysis of water BML-QDG-CL-004and specification.

- RO/WFI must be sampled from the sampling point everyday for chemical analysis.

Materials, Equipment, Apparatus and Reagents

- Conical flask (1000ml)

- Sampling Basket

- REFERENCE/ENCLOSURE

In House

- TRAINING INVOLVEMENT

Quality Manager

Quality control staff

All sectional heads in the quality department

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |