- OBJECTIVE:

To provide written guidelines for cleaning of glassware.

- SCOPE:

This SOP covers the process of cleaning glassware used in the chemical laboratory

3.0 RESPONSIBILITY/ACCOUNTABILITY:

- The Chemical analyst shall carry out the procedures using standard weights.

- The user shall ensure the equipment is kept clean and reports any breakages.

- The Quality Control Head ensures strict adherence to the procedure.

4.0 ABBREVIATIONS AND DEFINITIONS

- v/v; volume per volume

- Glass wares are all the apparatus made of glass that are used in analytical procedures

- Precision glass wares: glass wares calibrated to precision e.g. volumetric flask and bulb pipettes

- PROCEDURE

Glass wares used for water soluble solutions

- Discard solution in the fume hood for obnoxious solutions

- Rinse with potable water till the obnoxious smell is out

- With liquid soap of 250ml in 10 litres of purified water, wash with soft brush

- Rinse with potable water till soap is completely out

- Further rinse with purified water

- Then put the glassware to drain and air dry on the rack.

Glass wares used for water Insoluble Solution

- Discard solution in the fume hood for obnoxious solutions

- Rinse 2-3 times with 70% ethanol or acetone

- Rinse 4-5 times with potable water

- With liquid soap of 250ml in 10 litres of purified water, wash with soft brush

- Rinse with potable water till soap is completely out

- Further rinse with purified water

- Then put the glassware to drain and air dry on the rack.

Burettes

- Rinse thoroughly with potable water

- Discard solution in the fume hood for obnoxious solutions

- Rinse 2-3 times with 70% ethanol or acetone

- Rinse 4-5 times with potable water

- With liquid soap of 250ml in 10 litres of purified water, wash with soft brush

- Place burette upside down, on the burette stand, to dry

Pipette

- Run potable water, if used for a water soluble solution or a dissolving solvent, if used for a water insoluble solution through the pipette

- Run through pipettes with liquid soap of 250ml in 10 litres of purified water

- Rinse with potable water till soap is completely out

- Further rinse with purified water

- Then put the pipette rack to drain and air dry.

CAUTIONS

- Always remove stoppers and stopcocks when they are not in use. Otherwise, they may 'freeze' in place.

- Do not wash precision glass wares (bulb pipettes, volumetric flasks etc) with hot water

- Most new glass wares are slightly alkaline and should be washed upon receipt and generally can be soaked in a 1% v/v Hydrochloric acid or Nitric acid solution before washing and rinse.

- Never soak for long periods in strong alkaline solutions as it will damage the glass.

- Always use soft brushes or sponge to avoid abrasion.

- A detergent such as non-abrasive dishwasher mild solution should always be use.

- Discard left over sample in the glass wares

- Rinse with potable water

- Rinse thoroughly with plenty of clean tap water to remove all traces of soap.

- Further rinse with purified water

- Arrange in their respective cupboards away from dust.

- Discard broken glass wares appropriately

Materials, Equipment, Apparatus and Reagents

- PPE

- 70% ethanol or acetone

- 1% v/v Hydrochloric acid or Nitric acid solution

- liquid soap of 250ml in 10 litres of purified water

Safety, Health and Environment Precautions

- Wear appropriate PPE

- REFERENCE/ENCLOSURE

- Nil

- TRAINING INVOLVEMENT

| Quality Control Head | Chemical Analyst |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORY |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |