- PURPOSE:

- To lay down the procedure for using a respiratory mask.

- SCOPE:

- To provide written guidelines for using respiratory mask

- RESPONSIBILITIES/ACCOUNTABILITIES:

- It is the responsibility of any qualified personnel

- It is the responsibility of Biomedical Limited to provide the PPE for the qualified employee performing the task

- It is the responsibility of contactors to provide the PPE for their qualified staff

- It is the responsibility of the Engineering head to ensure that the SOP is strictly followed

- ABBREVIATION AND DEFINITIONS:

- SOP: Standard Operating Procedure.

- PPE: Personal Protective Equipment.

- GMP: Good Manufacturing Practices.

- PROCEDURE:

- Click on the tip of filters to the main filters, ensuring the filters are well attached to the mask

- Line up the filters with the twist lock mechanism on each side of the mask, press gently while locking clockwise until you feel the lock into place.

- Secondly, place the seal of the crown of the filters on your head, ensuring there is no twist in the straps. Glide the straps with one hand and put the mask up to your face with the other hand.

- Using both hands, connect the two straps behind your head.

- Adjust the straps to get a proper fit.

- Every time you are making use of a respiratory mask, ensure you perform a positive and negative fit check to ensure you have a proper face seal.

- To perform a negative pressure fit check:

Block the filters with the palm of your hand, if you feel air leaking between your face and the mask; adjust the mask and re-tightening the straps, before repeating the fit check. If properly sealed, your hands should collide slightly when put towards the face with no air leakage between the mask and the face.

- To perform a positive pressure fit check:

Place the palm of your hand over the exhalation valve and blow gently. If you have a proper seal, you should feel the mask bulge slightly with no air escaping between the mask and the face.

- REFERENCES AND ENCLOSURE:

- Nil

- TRAINING INVOLVEMENT:

All Engineering staff

All machine Operators

- DISTRIBUTION AND CONTROL COPY:

- This is the various department/section where the SOP copies will be utilized.

| MASTER COPY | QUALITY |
| --- | --- |
| CONTROLLED COPY | QUALITY, ENGINEERING, PRODUCTION |

- CHANGE HISTORY:

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 01 | 00 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |