- OBJECTIVE:

- This procedure serves as a guide for water sampling and analysis.

- To determine the effectiveness of the water treatment facilities.

- To ensure that water samples are collected in a manner that will be devoid of external contamination

- SCOPE:

- This procedure should be performed twice a week and whenever the plant is shut down before the commencement of operations.

- It is applicable to water from the ultra-filtration unit and storage Tank.

3.0 RESPONSIBILITY/ACCOUNTABILITY:

- The Microbiologist/Assistant is responsible for following the procedures outlined in this SOP

- The Quality Control Head is to ensure the procedures outlined are followed.

4.0 ABBREVIATIONS AND DEFINITIONS

- Nil

- PROCEDURE

## Sampling Procedure

- Ensure that hand gloves and nose mark are worn prior to sampling.

## Clean the tap and sanitize by swabbing with cotton wool soaked in 70% ethanol (or by flaming where possible for metal tap).

## The sampling point should be locked.

- Open the tap fully and allow it to flow for 2-3 minutes before sampling.

- The sterile sample bottle should be opened momentarily, and sample taken up to ¾ of its volume quickly to avoid any contamination from the neighborhood.

- Do not allow the ‘bottle’ or the ‘water in the bottle’ to come in contact with the valve/tap.

- Remove the bottle from the sample stream and place the cap on the bottle as quickly as possible

- Close the valve

- The sample should be taken to the laboratory and labeled with the sample point

- Water sample must be tested within 2 hours of sampling (or later than that, should be kept in the refrigerator).

- Water analysis test is carried out at syrup microbiology laboratory after appropriate line clearance done

REPORTING:

- The result should be recorded in water analysis sheet as per STP on water analysis BML/STP/RMA/054

- The result should be reported to QAM

- If the specification is exceeded, there is need for re-sanitization and this should be reported to the appropriate production personnel.

- REFERENCE/ENCLOSURE

- Standard Testing Procedure on Water Analysis - BML/STP/RMA/054

- TRAINING INVOLVEMENT

| Quality Control Head | Quality control microbiologist, Laboratory assistant |
| --- | --- |

8.0 DISTRIBUTION AND DOCUMENTATION CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | QUALITY CONTROL LABORATORIES |

9.0 REVISION HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| ### 00 | ### Nil | First issue |
| ### 02 | ### 01 | To Update |
| ### 03 | ### 02 | - Inclusion of enclosures format - Change in sop format generally |