- OBJECTIVE

To define the standard procedure for the safe and effective decontamination of reference organisms after usage in microbiological analysis, ensuring laboratory biosafety and compliance with pharmaceutical industry standards.

- SCOPE

This procedure applies to all microbiology laboratory personnel involved in the handling, processing, and disposal of reference organisms used for quality control testing within microbiological Laboratory environment.

- RESPONSIBILITIES/ACCOUNTABILITIES

- Microbiologists: To perform decontamination procedures as outlined.

- Quality Control Head: To ensure compliance with the SOP and conduct periodic reviews.

- Quality Assurance Manager: To audit and verify decontamination practices.

- ABBREVIATIONS AND DEFINITIONS

- PPE: Personal Protective Equipment

- IPA: Isopropyl Alcohol

- MSDS: Material Safety Data Sheet

- ATCC: American Type Culture Collection

- Reference Organisms: Standard microbial strains used as controls in microbiological testing (e.g., Escherichia coli ATCC 8739, Staphylococcus aureus ATCC 6538 etc.).

- Decontamination: The physical or chemical process of removing or neutralizing contaminants, such as microorganisms, to render items safe for handling or disposal.

5.0 PROCEDURE

Preparation before Decontamination

- Ensure all work involving reference organisms is completed.

- Put on appropriate PPE: lab coat, gloves, mask, and face shield.

- Transfer used culture plates, broth tubes, and other contaminated materials to the decontamination zone in a leak-proof, labeled biohazard container.

Decontamination of Solid Waste (e.g., agar plates, swabs)

- Autoclave at 121°C for 30 minutes under 15 psi pressure.

- After autoclaving, dispose of waste in accordance with local biohazard waste regulations such as NESREA/KWEPA

- Record autoclaving cycle parameters in the decontamination logbook.

Decontamination of Liquid Waste (e.g., broth cultures)

- Add an appropriate volume of 5% sodium hypochlorite solution (final concentration should be ≥1%) to liquid cultures.

- Allow contact time of at least 30 minutes.

- Dispose of neutralized liquids via laboratory drain with copious running water, if permitted by local regulations.

Decontamination of Equipment and Surfaces

- Clean work surfaces with 70% IPA or a validated disinfectant (e.g., peracetic acid-based solution).

- Rinse with sterile water if required.

- Autoclave or disinfect any reusable equipment (e.g., glassware, pipettes) before re-use.

Post-Decontamination Practices

- Wash hands thoroughly after decontamination.

- Document all decontamination steps performed, including date, time, reference organisms involved, and personnel name.

SAFETY

- Always wear appropriate PPE.

- Handle disinfectants with care; refer to corresponding MSDS for hazard information.

- Follow spill management procedures BML-QDG-QA-121 if accidental exposure or spill occurs.

6.0 REFERENCE DOCUMENTS

- WHO Guidelines on Good Manufacturing Practices for Pharmaceutical Products

- Biosafety in Microbiological and Biomedical Laboratories (BMBL)

- Material Safety Data Sheets (MSDS) of disinfectants used

7.0 TRAINING INVOLVEMENT

| Lab Assistant | Microbiologists |
| --- | --- |
| QC Head | QAM |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | MICRO LABS |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |