- OBJECTIVE:

To provide the method for product to product cleaning of all production equipment.

- SCOPE:

To provide written guidelines for product to product cleaning of all production equipment.

- RESPONSIBILITIES/ACCOUNTABILITIES:

Operators, Production Pharmacist and Q.A Officer

- ABBREVIATION AND DEFINITIONS

IPA – Isopropyl Alcohol

- PROCEDURES

- Weighing Balances

  - Clean the platform of the weighing balance with cloth to remove spilled powders of the previous batch. Also clean the display board and balance platform with lint free duster moistened with disinfectant

  - Clean the dispensing spoons and scoops with hot distilled water and a soft brush. Rinse three times with distilled water

- Mixing Tanks

  - Ensure that the mixing vessel is empty and close down the valves. Allow the distilled water to flow into the empty mixing tank, collect in a bucket and use to rinse the inside walls of the mixing tank thoroughly then drain the water from the tank

  - Ensure that the stirrer and mixing tank covers are cleaned properly

  - Drain the water from the tank

  - Repeat steps one more time

- Filtration and filling unit

  - Dismantle the filtration unit and remove filters. Clean the filtration unit with hot distilled water and fix it again without membrane filters

  - Fill the mixing tank with some hot distilled water and flush it through the filtration unit and filling nozzles continuously for 5 minutes

  - Dismantle the filter holding unit and fix the membrane filters.

  - Pass hot water continuously for 2 minutes then drain it. Run the transfer pump of the filtration unit continuously to remove all the water in the pipe until there is no more water emitting from the nozzles.

- Autoclave

  - Ensure that there aren’t any pouches from the previous batch in the autoclave

  - Make sure that the autoclaves are properly and correctly cleaned during the week

- Leak testing, Visual Inspection and Overwrapping Machine

  - Clean every part of the machine with napkin moistened with IPA before putting on the machine.

  - Clean the visual inspection table with a cloth to remove dirt

  - Also clean the packaging table with a clean cloth moistened with IPA

  - Ensure that there is no left over product materials from the previous product lying about packaging section.

Materials, Equipment

NA

- REFERENCE AND ENCLOSURE

In-House

7.0 TRAINING INVOLVEMENT

| All departmental managers | All sectional heads |
| --- | --- |
| QA Doc. Unit staff | All staff |

8.0 DISTRIBUTION AND CONTROL:

| MASTER COPIES | QUALITY ASSURANCE |
| --- | --- |
| CONTROLLED COPIES | ALL DEPARTMENT |

9.0 CHANGE HISTORY

| CURRENT REVISION | SUPERSEDE | REASON FOR REVIEW |
| --- | --- | --- |
| 00 | Nil | First issue |
| 06 | 05 | - Change in sop format. - Inclusion of traceability of enclosures to parent SOP |